Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EC2066 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | DIGITAL SYSTEM DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Prove the following identity using exhaustion method. A′C + CD + AB′C′ = A′C + CD + A B′ C ′ + ACD | CO1 | 4 |
|  | b. | Develop a circuit using combinational logic for 4-bit Gray to Binary code converter by considering D,C,B,A as inputs and W,X,Y,Z as outputs. | CO1 | 10 |
|  | c. | Create the truth table and draw the logic diagram for the system specified by following Boolean equation. F= A'B'C + A'BC + AB'C G= A'BC' + A'BC + ABC' | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. | a. | A system has 4 inputs and is to produce an output which is TRUE if and only if an odd number of inputs are TRUE. Design a logic circuit that will meet this specification. | CO1 | 8 |
|  | b. | Use 2 half-adders to realize a full adder. | CO1 | 4 |
|  | c. | Minimize the following logic function using K-map Y (A,B,C,D) =∑m (0,1,2,3,5,7,8,9,11,14) and implement it using logic gates. | CO1 | 8 |
| 3. | a. | Design a synchronous sequential machine for the overlapping input sequence detector “1010”sequence for Moore state machine. | CO1 | 14 |
|  | b. | Differentiate Mealy and Moore machine and draw the general model for Mealy Sequential design. | CO1 | 6 |
| (OR) |  |  |  |  |
| 4. | a. | Design a synchronous sequential machine for the overlapping input sequence detector “111”sequence for Mealy state machine. | CO1 | 12 |
|  | b. | Reduce the following function using K-map technique and implement using basic gates: Y (A,B,C,D)= A'B'D + ABC'D' + BCD | CO1 | 8 |
| 5. | a. | Design a BCD to excess-3 code converter using a suitable PLA. | CO2 | 10 |
|  | b. | Design an asynchronous sequential circuit to behave in the following manner. The circuit has two inputs x1 and x2 and two outputs P and Q. Initially both inputs and outputs are equal to 0 (state 0).Also if x1= x2= 1,P=Q=1 (state 4). When x2 changes from 0 to 1,P becomes 1 and Q becomes 0 for state 2.Since the second input x2 is 1 Q becomes 1 and P takes the initial state value i.e 0 for state 3.When x1=1 and x2=0 , P becomes 1 and Q becomes 0 for state 5 and Q becomes 1 and P becomes 0 for state 6. Whenever, P=0 and either of x1 or x2 becomes 1,P becomes 1.When the second input becomes 1, Q also becomes 1. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Design a suitable PAL for the following function: W(A,B,C,D)=∑m (0,2,6,7,8,9,12,13) X(A,B,C,D)= ∑m (0,2,6,7,8,9,12,13,14) Y(A,B,C,D)= ∑m (2,3,8,9,10,12,13) Z(A,B,C,D)= ∑m (1,3,4,6,9,12,14) | CO2 | 10 |
|  | b. | Design an asynchronous circuit that will output only the first pulse received and ignore any other pulse. | CO2 | 10 |
| 7. | a. | With neat block diagrams explain in detail about State Box, Decision Box and Conditional Box in ASM chart. | CO2 | 8 |
|  | b. | With Embedded Array block explain in detail about device block diagram of ALTERA FLEX 10K CPLD. | CO2 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | Define ASM chart and draw the ASM for 3-bit binary up/down counter. | CO2 | 8 |
|  | b. | Design a suitable PLA for the following function with PLA table. F0(A,B,C)=∑m (0,1,4,6) F1(A,B,C)= ∑m (2,3,4,6,7) F2(A,B,C)= ∑m (0,1,2,6) | CO3 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | With neat block diagram explain in detail about ALTERA FLEX 10K CPLD logic element. | CO2 | 14 |
|  | b. | Implement a full adder using a PAL. | CO3 | 6 |