Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2067 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # VERILOG HDL | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Outline in detail about the Integer, Real and Time Register Data types in Verilog with suitable examples. | CO2 | 8 |
|  | b. | Describe about 8X1 mux and write the behavioral modeling (using CASE Statement) using Verilog. | CO2 | 6 |
|  | c. | Show the post declarations of 4-bit adder and D-FF, and list the various type of Port in Verilog. | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. | a. | Illustrate in detail about the two types of Number Specification. | CO1 | 8 |
|  | b. | Review about the design of full subtractor and write the gate-level modeling using Verilog. | CO2 | 6 |
|  | c. | Discuss about the data types including Value Set,Nets,Registers and Data Types in Verilog. | CO2 | 6 |
| 3. | a. | Illustrate a Verilog code for a counter which increments the count from 0 to 127 and exit at count 128 using while loop. | CO3 | 2 |
|  | b. | X=4’b0011; Y=4’b1011;Z=4’b x011; Calculate : X & Y X^~Y X&Z X^Y | CO1 | 2 |
|  | c. | Perform reduction operation: A=4’b0101; B=4’b0110; ¦ B, &B, ~ ^ A, ¦ A | CO1 | 2 |
|  | d. | Illustrate about the different types of Gate delays. | CO1 | 4 |
|  | e. | Design a 4-bit ripple counter using negative edge triggered flip-flops. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Write the Verilog program using IF statement for T-Flip-flop with positive edge triggered clock. | CO2 | 5 |
|  | b. | Design a 4X1 multiplexer and write Verilog Code using Conditional Operators. | CO2 | 6 |
|  | c. | Describe about Continous assignments in Verilog and also write the instantiation statement for 2-input AND Gate. | CO2 | 9 |
| 5. | a. | Summarise in detail about Bidirectional Switches, Power and Ground used in switch level modeling. | CO2 | 8 |
|  | b. | Design switch level modeling of CMOS NOR Gate. | CO2 | 4 |
|  | c. | Tabulate the different operators used in Verilog. | CO2 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | Summarise in detail about MOS Switches ,CMOS Switches and Resistive Switches. | CO2 | 10 |
|  | b. | Develop a Switch-Level Verilog Description of CMOS Flip-flop. | CO3 | 10 |
| 7. | a. | Examine about always and initial statement in behavioral modeling. | CO2 | 12 |
|  | b. | Design Z=(A.B)+(C.D) using CMOS Logic and write switch level Verilog Description. | CO3 | 8 |
| (OR) |  |  |  |  |
| 8. |  | Design Finite State Machine for Newspaper Vending Machine and write the Verilog Description. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | With neat examples describe about AND/OR and Buf/NOT Gates under Gate Modeling. | CO2 | 12 |
|  | b. | Describe about test bench and develop the test bench for 4-to-1 Multiplexer. | CO3 | 8 |