Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2072 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ANALYSIS AND DESIGN OF DIGITAL IC | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define Channel region in MOSFET. | CO2 | 2 |
|  | b. | Sketch the physical structure of nMOS transistor and its schematic icon. | CO2 | 6 |
|  | c. | With neat diagram, illustrate the different regions of operation of NMOS transistor. | CO2 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | How would you obtain the voltage transfer curve of CMOS inverter? | CO2 | 4 |
|  | b. | Indicate the value of Y in Fig 1a and 1b when A=1 and B=0. Justify the same. Fig 1a Fig 1b | CO2 | 6 |
|  | c. | With the voltage transfer curve of CMOS inverter indicate how to calculate NML and NMH. Analyze the impact of βn/βp ratio in the VTC of CMOS inverter. | CO2 | 10 |
| 3. | a. | Justify the statement “NMOS transistors pass strong zero but not strong one”. | CO2 | 6 |
|  | b. | Develop the analytical delay model of a CMOS inverter to describe its switching characteristics. | CO2 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | Indicate with equations, how to identify the region of operation of a NMOS transistor. | CO2 | 6 |
|  | b. | With the voltage transfer curve of CMOS inverter indicate how to calculate NML and NMH. Analyze the impact of βn/βp ratio in the VTC of CMOS inverter. | CO2 | 14 |
| 5. | a. | Design a 2 input AND gate using complementary CMOS and pseudo NMOS design styles and reason out which design style is efficient. | CO1 | 6 |
|  | b. | With diagram indicate the different issues that occur in dynamic logic design style and provide suitable solutions for the same. | CO3 | 14 |
| (OR) |  |  |  |  |
| 6. | a. | Design a 2 input AND gate using complementary CMOS and pass transistor logic design styles and reason out which design style is efficient. | CO1 | 6 |
|  | b. | With mathematical equations, illustrate the different types of power dissipation that occurs in CMOS inverter design. | CO3 | 14 |
| 7. | a. | Sketch the timing diagram of negative latch. | CO3 | 2 |
|  | b. | With block diagram differentiate between combinational and sequential logic circuits. | CO3 | 4 |
|  | c. | Design F=ABC using np-CMOS logic design style and explain its operation. | CO1 | 14 |
| (OR) |  |  |  |  |
| 8. | a. | Draw the block diagram of MUX based positive latch. | CO3 | 2 |
|  | b. | Differentiate between static and dynamic memory with respect to its storage and clocking. | CO3 | 4 |
|  | c. | Design and explain the operation of positive edge triggered register with timing diagram and analyze the demerits of the design. | CO1 | 14 |
|  |  | Compulsory: |  |  |
| 9. | a. | Design a 2 input OR gate using true single phase clocked approach and explain its operation. | CO1 | 10 |
|  | b. | Design a pipelined datapath using C2MOS latches and mention the advantages of C2MOS latches over C2MOS registers. | CO1 | 10 |