Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EC2079 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MICROPROCESSORS AND MICROCONTROLLERS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | List the interrupts of 8085 microprocessor according to its priority. | CO1 | 5 |
|  | b. | Draw the timing diagram for LDA 4500H. Find the time required to execute the same if the clock frequency is 3MHZ. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | With block diagram explain various internal functions of 8085 microprocessor. | CO1 | 10 |
|  | b. | Give the addressing modes for the following instruction in 8085. MVI A, 12H HLT ADD M STA 4500 MOV B,A | CO1 | 10 |
| 3. | a. | # Justify the necessityfor portsin microcontroller. | CO2 | 5 |
|  | b. | Outline the features of 8051 and discuss its various functional blocks. | CO2 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Summarize the different groups of instructions supported by 8051. Explain them with suitable examples. | CO2 | 15 |
|  | b. | Distinguish Timer and Counter of 8051 microcontroller. | CO2 | 5 |
| 5. | a. | Calculate the amount of time delay when the timer TH0 is loaded with count value of 9C in mode2 operation. | CO2 | 5 |
|  | b. | Give the control logic operation in 8051 timer and explain its diiferent modes of operation. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | Write an assembly language program to find the product of two 8 bit numbers in 8051. | CO2 | 5 |
|  | b. | Describe the addressing modes of 8051 Microcontroller. | CO2 | 15 |
| 7. | a. | Design a 8051 system to interface stepper motor. | CO3 | 10 |
|  | b. | Demonstrate the data serialization using rotate instructions of 8051. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Draw the schematic to interface 8 LEDs with 8051 microcontroller through 8255 programmable peripheral interface. Write an assembly language program to glow the led. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Design a circuit to interface LCD display with 8051 microcontroller. Write an assembly language program to display “HELLO”. | CO3 | 20 |