Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASIC ELECTRICAL ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | State and explain Ohm's law. Mention its limitations. | CO1 | 6 |
|  | b. | Using Kirchhoff’s laws, find the current in various resistors in the circuit shown below: | CO1 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Determine the following quantities for the circuit given in below, i) Total Resistance RT; ii. Total current IT; iii. Power delivered by the voltage source. | CO1 | 6 |
|  | b. | Give the characteristics of series and parallel circuit. Also justify the reason for domestic appliances connected in parallel. | CO1 | 14 |
| 3. |  | # Find Vavg, Vrms, form factor, peak factor, frequency and time period for the sinusoidal voltage equation given below; # v = 230sin314t | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | State and explain the laws of Electromagnetic Induction with necessary equations. | CO1 | 8 |
|  | b. | Draw the one line diagram of typical power system. Mark all the components. | CO2 | 6 |
|  | c. | Bring out a table of comparison between overhead and underground cable system. | CO2 | 6 |
| 5. |  | With a neat diagram explain the electricity generation by thermal power plant. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | A 6 pole lap wound DC Generator has 1000 conductors. The flux per pole is 10 milliwebers. Determine the induced emf in the armature, if it is rotating at a speed of 600 rpm. | CO2 | 5 |
|  | b. | Derive the emf equation and explain the working principle and construction of a DC Generator with neat Diagram. | CO2 | 15 |
| 7. |  | Justify the necessity of the following torques in indicating instruments. - Deflecting torque. - Controlling torque. - Damping torque. | CO3 | 20 |
|  |  | (OR) |  |  |
| 8. | a. | Describe the working principle of a permanent magnet moving coil instrument with a neat sketch. | CO3 | 10 |
|  | b. | Explain the principle of operation of a transformer. What are its applications? | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Demonstrate the following with a neat wiring diagram. - Stair-case wiring. - Fluorescent tube connection. | CO3 | 20 |