Reg.No. ____________

End Semester Examination – April/ May– 2018

| # Code : | # 14EE2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRIC CIRCUITS AND NETWORKS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Determine the value of the equivalent resstance RAB. | CO1 | 12 |
|  | b. | Obtain a single equivalent voltage across the terminals A and B using source transformation technique. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Determine the current through the 2Ω and 6Ω resistors using mesh analysis | CO1 | 15 |
|  | b. | Explain with an example the source transformation technique. | CO1 | 5 |
| 3. |  | Determine the current through the 10Ω resistor using the Superposition theorem. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Obtain Thevinin’s equivalent circuit with respect to the terminals A and B | CO1 | 15 |
|  | b. | State and explain Norton’s theorem. | CO1 | 5 |
| 5. | a. | A series RLC circuit with R = 30 Ω, L = 80 mH and C = 80 µF has an applied voltage of 150 V, determine the value of (i) resonant frequency (ii) current at resonance (iii) Q-factor and (iv) Bandwidth. | CO2 | 14 |
|  | b. | State Dot rule for couple circuits. Use Dot rule to find the equivalent inductance of two coupled coils in series. | CO1 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | A 3 phase, balanced, delta connected load of 5+j10 ohm is supplied from a balanced, three phase supply of 400V (line to line). Determine the phase and the line currents. Also determine the power consumed by the circuit. | CO2 | 10 |
|  | b | Obtain a expression to find the optimum amplification factor of a single tuned coupled circuit. | CO1 | 10 |
| 7. |  | Obtain the transient current, voltage across the resistor and voltage across the capacitor of a series RC circuit when it is switched to a unit step voltage source at time t=0. Assume initial charge across the capacitor as zero.. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | In the circuit given, switch is closed at time t=0. Obtain the transient current voltage across the resistor and voltage across the inductor. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Obtain the short circuit admittance parameters for the T net work given below. | CO1 | 10 |
|  | b. | Design a constant K low pass filter having a cut off frequency of 1 KHz to operate with a terminated load resistance of 700 ohm. | CO3 | 10 |