Reg.No. ____________

End Semester Examination – April/May– 2018

| # Code : | # 14EE2004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTOMAGNETIC FIELDS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Find the dot product and the cross product of the vectors A=4ax−3ay+2az and B=3ax+ay+2az Also find the angle between the vectors . | CO1 | 10 |
|  | b. | State and explain Divergence theorem. | CO2 | 5 |
|  | c. | State and explain Stoke’s theorem. | CO2 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Given that F=2ax−ay+5az. Express the vector in spherical coordinate system at the point (1m,2m,1m). | CO1 | 12 |
|  | b. | Prove that the vectors A=4ax−2ay−az and B=ax+4ay−4az are perpendicular to each other. | CO1 | 8 |
| 3. | a. | Three point charges q1 = 10- 6 C, q2 = -10 -6 C and q3 = 0.5 x 106 C are located at A(0,0,0), B(3,1,0) and C(2,2,2) respectively. Determine the magnitude and direction of the force on q3. | CO2 | 10 |
|  | b | State Coulomb’s law and explain it with an example. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Use Gauss’s law to find the electric field intensity at any point due to an infinite surface charge with charge density ρsC/m2. | CO1 | 8 |
|  | b. | Derive an expression to find the electric potential and electric field intensity at any point due to an electric dipole. | CO1 | 12 |
| 5. | a. | Derive an expression to find the capacitance of a spherical capacitor and hence to determine the capacitance of a sphere of diameter 1 cm and relative permittivity 4. | CO1 | 10 |
|  | b. | Deduce the relation between electric flux density and electric field intensity. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | State Biot Savart’s law. Derive an expression to find the magnetic field intensity H at a distance ρ meter away from an infinite conductor carrying current I amps. | CO2 | 11 |
|  | b. | Determine whether or not following potential fields satisfy the Laplace’s equation. a. V=x2 –y2+z2 b. V=rcosφ +z c. V=rcos θ +φ | CO1 | 9 |
| 7. | a. | State and explain Ampere’s circuital ;law. Explain, how this law can be effectively used to determine the magnetic field intensity due to a co axial cable at various distances away from it. | CO2 | 14 |
|  | b. | Define current density. Obtain continuity equation for the current. | CO2 | 6 |
| (OR) |  |  |  |  |
| 8. | a. | Derive an expression to find the self inductance of a coaxial cable. | CO2 | 13 |
|  | b. | Calculate the inductance of a solenoid 8cm in length 2cm in radius having μr =100 and carrying 900 turns of wire. | CO1 | 7 |
|  |  | Compulsory: |  |  |
| 9. | a. | Obtain Maxwell’s equations in integral form and point form from the basic concepts. | CO3 | 15 |
|  | b. | Explain self inductance and mutual inductance from the practical point of view of a transformer. | CO2 | 5 |