Reg.No. ____________

End SemesterExamination – April/ May– 2018

| # Code : | # 14EE2004 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTOMAGNETIC FIELDS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Prove that the divergence of curl of vector is zero. | CO2 | 8 |
|  | b. | Compare the incremental lengths, incremental area and incremental volume of all the three coordinate system. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | Given that F=2ax−ay+5az. Express the vector in cylindrical coordinate system at the point (1m,2m,1m). | CO1 | 10 |
|  | b. | Use spherical coordinates and integrate to find the area of the region 0≤φ≤α on the spherical shell of radius a. What is the area if α=2π? | CO1 | 10 |
| 3. | a. | An infinite line charge with charge density ρl is placed along Z- axis. Derive an expression to find the electric field intensity at a distance ρ meter away from origin on X-axis. | CO2 | 10 |
|  | b. | A hollow sphere is charged to 10  C of electricity. Find the potential (a) at its surface (b) inside the sphere (c) at a distance 0.3m from the surface. The radius of the sphere is 0.1 m. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | State Gauss’s law and express it mathematically.Explain any one of its applications. | CO2 | 10 |
|  | b. | Three point charges q1 = 10- 6 C, q2 = -10 -6 C and q3 = 0.5 x 10 -6C are located at A(1,2,-3), B(1,1,0) and C(3,0,-1) respectively. Determine the magnitude and direction of the force on q3. | CO1 | 10 |
| 5. | a. | Derive an expression to find the capacitance of a co axial cable. | CO1 | 10 |
|  | b. | Derive Poisson’s and Laplace’s equations for electrostatic field. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | State BiotSavart’s law. Use the law to derive an expression to find the magnetic field intensity H at any point on the central axis of a circular current carrying loop carrying a current I amps in it. | CO1 | 12 |
|  | b | Derive the expression for torque acting on a rectangular loop carrying a current I. | CO1 | 8 |
| 7. | a | Find the inductance of an ideal solenoid with 300 turns. ℓ= 0.50m, and a circular cross section of radius 0.02m. | CO2 | 7 |
|  | b | An air core toroid has a mean radius of 40mm and is wound with 4000 turns of wire. The circular cross section of the toroid has a radius of 4mm. A current of 10A is passed in the wire. Find the inductance. | CO1 | 8 |
|  | c | State and explain point form of Ohms law. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Define and obtain expressions for transformer E.M.F. and motional E.M.F. | CO1 | 10 |
|  | b. | Obtain the magnetic boundary conditions between two magnetic materials. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive Maxwell’s equation in integral and point forms. | CO1 | 10 |
|  | b. | Obtain the wave equation through dielectrics. Describe about various parameters that define a wave. | CO3 | 10 |