Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DC MACHINES AND TRANSFORMERS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw the block diagram and explain electromechanical energy conversion. | CO1 | 6 |
|  | b. | With neat sketch, explain the doubly excited magnetic field system in electro mechanical energy conversion. Also obtain the expression for field energy. | CO1 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Give the relation between energy and co-energy for a linear system. | CO1 | 6 |
|  | b. | Draw the schematic showing a single excited electromechanical energy conversion system and derive the expression for energy stored in the magnetic field. | CO1 | 14 |
| 3. | a. | Draw and explain the no-load and load characteristics of dc shunt generator. | CO1 | 8 |
|  | b. | A 4-pole, lap wound, DC shunt generator has a useful flux per pole of 0.06Wb. The armature winding consists of 220 turns each of 0.004 ohm resistance. Calculate the terminal voltage and load current when it runs at 900rpm, if the armature current is 50A. (Rf= 200 ohm). | CO1 | 12 |
| (OR) |  |  |  |  |
| 4. | a. | Mention the effects of armature reaction. Explain the various methods of decreasing the effects of armature reaction. | CO1 | 14 |
|  | b. | A 450kW, 400V, 8-pole, lap wound DC generator has 768 armature conductors. If the brushes are given a lead of 6 electrical degrees, calculate the number of demagnetizing and cross-magnetizing ampere-turns per pole at full load. Shunt field current may be neglected. | CO1 | 6 |
| 5. | a. | Draw a neat sketch of a 4-point starter for a DC shunt motor and explain its working. Mention how the motor is protected against overloads and loss of supply. | CO2 | 14 |
|  | b. | In a DC machine, the total iron loss at the rated speed and excitation is 9kW. On keeping excitation unaltered but reducing speed by 30%, the total iron loss was found to be 5kW. Calculate the hysteresis and eddy current losses at full speed. | CO2 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Describe with the aid of a circuit diagram the Hopkinson’s test for a pair of DC shunt machines. State the advantages and disadvantages of this method. | CO3 | 14 |
|  | b. | Two shunt machines loaded for the Hopkinson’s test take 15A at 220V from the supply mains. The generator output current is 85A and the shunt field circuit currents are 3A and 2.5A, resistance of the armature is 0.05 ohm. Calculate the efficiency of generator at this particular load condition. | CO3 | 6 |
| 7. | a. | Draw and explain the phasor diagram of a single phase transformer supplying: a. a lagging p.f load b. a leading p.f load c. a unity p.f. load | CO3 | 16 |
|  | b. | Define all-day Efficiency of a transformer. Mention the various losses of the transformers. | CO2 | 4 |
| (OR) |  |  |  |  |
| 8. |  | The parameters of a 2300V/230V, 50Hz transformer are R1=0.3Ω, R2'=0.295Ω, Rl1=4000Ω, X1=0.7Ω, X2'=0.685Ω, Xm1=1000Ω. The secondary load impedance ZL=(0.4+j0.3) ohm. Find the performance of the transformer using exact equivalent circuit. The transformer is supplied at the rate voltage. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Brief about Sumpner’s test on transformer. | CO3 | 10 |
|  | b. | Derive an expression for the saving in copper effected by using an auto transformer instead of a two winding transformer. | CO3 | 10 |