Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INDUCTION AND SYNCHRONOUS MACHINES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Enumerate the role of induction motor used in textile mill drives. | CO1 | 5 |
|  | b. | The power input to a 400 V, 50 Hz, 4 Pole, three phase induction motor running at 1440 rpm is 40kW at 0.86 p.f. lag. Stator losses are 1 kW and friction and windage losses are 2 kW. Calculate slip, rotor copper loss, brake hp, efficiency and input current. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | Sketch the power flow diagram of asynchronous motor. | CO1 | 5 |
|  | b. | Identify the type of motor are shown below. | CO1 | 5 |
|  | c. | Calculate the number of poles required, when the frequency is 50 Hz and speed of the motor is 500 rpm. | CO1 | 10 |
| 3. |  | # A 400 V, 40 hp, 50 Hz, 4-pole, delta connected induction motor gave the following data. # No load test: 400 V, 20 A, 1200 W # Blocked rotor test: 100 V, 45 A, 2800 W Draw the circle diagram and determine the line current, power factor for rated output, maximum output, maximum torque and full load efficiency. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | List out the applications of 3-phase squirrel cage induction motors. | CO1 | 2 |
|  | b. | Sketch the torque – slip characteristics of an induction motor. | CO1 | 5 |
|  | c. | Outline the concept of dynamic braking. | CO1 | 5 |
|  | d. | A 12 pole, 3-phase alternator is coupled to an engine running at 550 rpm. It supplies an induction motor which has a full-load speed of 1440 rpm. Compute the slip of the motor. | CO1 | 5 |
|  | e. | Express distribution factor. | CO1 | 3 |
| 5. | a. | Express the power development stages in an induction motor and obtain P2 : Pm : I2R loss = 1 : (1-s) : s | CO2 | 15 |
|  | b. | Realize the concept of cogging and crawling. | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | The stator of a 3ϕ, 16 pole alternator has 144 slots and there are 4 conductors per slot connected in two layers and the conductors of each phase are connected in series. If the speed of the alternator is 375 rpm, calculate the emf induced per phase. Resultant flux in the airgap is 5 × 10-2 Wb per pole sinusoidally distributed. Assume the coil span as 150 deg elect. | CO2 | 15 |
|  | b. | Elaborate the principle of operation of linear induction motor used in locomotive. | CO2 | 5 |
| 7. | a. | Formulate the expression of linear force in LIM. | CO2 | 5 |
|  | b. | An alternator is generating power at 210 V per phase while running at 1500 rpm. If the need of the alternator drops to 1000 rpm, find the generated voltage per phase. | CO2 | 5 |
|  | c. | Define repulsion motor. | CO3 | 4 |
|  | d. | Predict the type of single phase induction motor would you use for ceiling fan and wet grinder and explain in detail. | CO3 | 6 |
| (OR) |  |  |  |  |
| 8. | a. | List the various modes of excitation in stepping motors. | CO3 | 4 |
|  | b. | Sketch the phasor diagram of alternator for resistive, Inductive and capacitive Load by using EMF method. | CO3 | 9 |
|  | c. | Name the winding types of Schrage motor used in Textile mills. | CO3 | 3 |
|  | d. | Identify the type of motors are shown below. | CO3 | 4 |
|  |  | Compulsory: |  |  |
| 9. | a. | A 60 kVA, 220V, 50 Hz, single phase alternator has effective resistance of 0.016Ω and an armature leakage reactance of 0.07 Ω. Compute voltage induced in the armature when alternator is delivering rated current at load power factor of i) UPF ii) 0.7 P.F. lagging iii) 0.7 P.F. leading. | CO3 | 10 |
|  | b. | Find the maximum speed of a 2 pole, 50 Hz alternator. | CO3 | 4 |
|  | c. | Explain V curve and Inverted V curve of a synchronous motor. | CO3 | 6 |