Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2009 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRICAL MACHINE DESIGN | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the methods to calculate the MMF for teeth. | CO1 | 10 |
|  | b. | A 15 kW, 250V, 4 pole dc machine has the following data: armature diameter = 0.25m, armature case length = 0.125m, length of air gap at pole centre = 2.5mm, flux per pole is 11.7mWb, ratio of pole arc to pole pitch is 0.66. Calculate mmf required for air gap if the armature surface is smooth and gap contraction factor is 1.18. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Derive the output equation for a DC machine. | CO1 | 10 |
|  | b. | Determine the airgap length of a dc machine from the following particulars. Gross length of core =12cm, number os ducts =1 and is 10mm wide, slot pitch =25mm, slot width=10mm. Take carter’s co-efficient for slots and ducts=0.32. Gap density at pole centre =0.7Wb/m2. Field mmf/pole =3900AT, mmf required for iron parts of magnetic circuits=800AT. | CO1 | 10 |
| 3. | a. | A shunt field coil has to develop an mmf of 9000 AT. The voltage drop in the coil is 40V, resistivity of the round wire is 0.021 Ω / m – mm2. The depth of winding is 35mm and length of mean turn is 1.4m. Design a coil so that the power dissipated is 700 W/m2 of the total coil surface. Take diameter of the insulated wire 0.2 mm greater than that of bare wire. | CO2 | 15 |
|  | b. | What are the advantages of having more number of poles in a DC machine? | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | What the factors affects the length of airgap length of dc machine? | CO2 | 5 |
|  | b. | Determine suitable values for the number of poles, diameter, core length of a dc motor with following data. 20HP, 1000 rpm, 400V, specific magnetic loading 0.37 Tesla and specific electric loading 16000 ac/m, efficiency 90%, assume square pole face. | CO2 | 15 |
| 5. | a. | Derive the output equation of single phase and three phase transformer. | CO3 | 10 |
|  | b. | A 250kVA, 6600/400V, 3 phase core type transformer has a total loss of 4800W on full load. The transformer tank is 1.25m in height and (1×0.5)m in plane. Design a suitable scheme for cooling tubes if the average temperature rise is to be limited to 350C. The diameter of tube is 50mm and are spaced 75mm from each other. The average height of tube is 1.05m. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Draw the 2 stepped core and obtain the expression for the net core area in terms of square of diameter of circumscribing circle. | CO3 | 8 |
|  | b. | Determine the main dimensions of the window, core, number of turns and the cross section of the conductors for a 5 kVA, 1100/400V, 50Hz, single phase core type distribution transformer. The net conductor area in the window is 0.6 times the net cross section of iron in the core. Assume a square cross section of the core, a flux density 1 wb/m2, current density 1.4 A/ mm2 and window space factor 0.2, Height of window is 3 times its width. | CO3 | 12 |
| 7. | a. | What are the factors affects the length of airgap of induction machine? | CO2 | 5 |
|  | b. | Determine the approximate length and diameter of stator core, the number of stator slots and the number of conductors for a 20kW, 400V, 3 phase, 4 pole, 1200 rpm, delta connected induction motor. The specific magnetic loading is 0.5T, efficiency is 0.82, ac = 26000amp.conductors/m, power factor = 0.8, L/τ = 1. | CO2 | 15 |
| (OR) |  |  |  |  |
| 8. |  | A 90kW, 500V, 50Hz, 3 phase, 8 pole induction motor has a star connected stator winding accomadated in 63 slots with 6 conductors per slot. If the slip ring voltage on open circuit is to be about 400V. Find a suitable rotor winding stating: - Number of rotor slots. - Number of conductors per slot. - Coil span. - Approximate full load current per phase in rotor. - Cross sectional area of rotor conductor. Assume efficiency 0.9 and power factor 0.86. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Define Short Circuit Ratio. What are the significance of Short Circuit Ratio. | CO2 | 5 |
|  | b. | The armature of a 10 pole, 1000kW, 500V, 300rpm, dc generator has a diameter of 1.6m. There are 450 coils. Determine suitable axial length and diameter of commutator giving details of brushes, having regard to commutation conditions and temperature rise. The design limitations are: peripheral speed of commutator 20m/s, pitch of segments 4mm, current/brush 70A, temperature rise 400C. The other data given is: The brushes span 3 segments approximately; brush drop=1.5V, co-efficient of friction =0.012. Make suitable assumptions for clearance between brush boxes, staggering of brushes and end play. | CO3 | 15 |