Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER ELECTRONICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | With neat diagram describe the switching characteristics of IGBT. | CO1 | 10 |
|  | b. | Compare Power MOSFET and Power BJT. | CO1 | 5 |
|  | c. | What are the protection mechanisms used to protect SCR from over voltage and over currents? | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the Static and switching characteristics of Thyristor. | CO1 | 15 |
|  | b. | What is snubber circuit? Mention its significance. | CO1 | 5 |
| 3. | a. | Sketch the circuit diagram of three phase half wave controlled rectifier with R load and explain its working. | CO2 | 6 |
|  | b. | A single phase fully controlled full bridge converter is supplied by 230V, 50Hz. It is connected with R-L Load. - Determine the average and rms output voltage if the firing angle is 45°. - Calculate the firing angle for which the average output voltage of the converter is 100V. | CO2 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | Compare single phase full and semi controlled converters. | CO3 | 5 |
|  | b. | Explain the operation of a single phase semi converter bridge for resistive load with neat diagram and waveforms. | CO2 | 15 |
| 5. |  | Analyze the working of single phase to single phase step down cyclo-converter with circuit diagram and waveforms with R load. Also obtain the expression for rms output voltage. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Explain four quadrant chopper with relevant diagrams. | CO2 | 10 |
|  | b. | Draw the circuit diagram and waveform of single phase unidirectional AC-AC converter with R load. | CO2 | 5 |
|  | c. | A step down chopper has Vdc = 200 V, load resistance R = 10 Ω, switching frequency is 1kHz. If the average output voltage is 120V, Calculate the duty cycle and ON time period TON. | CO2 | 5 |
| 7. |  | Describe the operation of three phase inverter in 120 mode conduction with necessary circuit diagram and waveforms. Write the expression for the rms value of phase voltage and line voltage. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | The single phase full bridge inverter has a resistive load of R=2.4Ω and the dc input voltage is Vs=48V. Determine (i) rms output voltage (ii) the rms output voltage at the fundamental frequency (iii) the output power. | CO3 | 6 |
|  | b. | Explain about the working of single phase full bridge inverter with RL load with relevant circuit diagram and waveforms. Also obtain the expression for the rms output voltage. | CO2 | 14 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the working of UPS with neat circuit diagram. | CO3 | 10 |
|  | b | With circuit diagram and waveforms explain the operation of phase controlled rectifier fed DC Motor Drive. | CO2 | 10 |