Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code :14EE2013 |  | Duration | :3hrs |
| --- | --- | --- | --- |
| Sub. Name : TRANSMISSION AND DISTRIBUTION |  | Max. Marks | :100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A generating station has the following daily load cycle Time (Hrs); 0-6; 6-10; 10-12; 12-16; 16-20; 20-24. Load (MW); 50; 60; 70; 50; 80; 50. Draw the following - i) Load curve ii) Load Duration Curve iii) Maximum Demand iv) Units Generated/Day iv) Average Load v) Load Factor. | CO1 | 10 |
|  | b. | Derive an expression for two phase four wire system. |  | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Sketch a typical Alternating Power Supply System and mark the voltage levels from generation, through transmission up to distribution systems. | CO1 | 10 |
|  | b. | Time; Consumer 1; Consumer 2; Consumer 3. 12 midnight to 8 A.M; No Load; 200W; No load. 8 A.M to 2 P.M; 600 W; No Load; 200 W. 2 P.M to 4 P.M; 200W; 1000 W; 1200 W. 4 P.M to 10 P.M; 800 W; No Load; No Load. 10 P.M to midnight; No Load; 200 W. The daily demands of three consumers are given below: Plot the load curve and find i) maximum demand of individual consumer ii) load factor of individual consumer iii) diversity factor iv) load factor of the station | CO1 | 10 |
| 3. | a. | Deduce an expression for inductance per phase for a 3-phase overhead transmission line when i)Conductors are symmetrically placed ii) Conductors are unsymmetrically placed but the line is completely transposed. | CO2 | 15 |
|  | b. | Calculate the capacitance of a 100 km long 3-phase, 50 Hz overhead transmission line consisting of 3 conductors, each of diameter 2 cm and spaced 2·5 m at the corners of an equilateral triangle. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Show how regulation and transmission efficiency are determined for medium lines using nominal T method . | CO2 | 5 |
|  | b. | A 3-phase, 50Hz transmission line 100 km long delivers 10 MW at 0.8 pf lagging and at 110 kV. The resistance and reactance of the line per phase per km are 0.2 ohm and 0.4 ohm respectively, while capacitance admittance is 2.5 x 10-6siemen / km/ phase. a. Calculate the current and voltage at the sending end. b. Efficiency of transmission. Use Nominal T method | CO2 | 15 |
| 5. | a. | Derive the expression for string efficiency for suspension type insulator. | CO2 | 10 |
|  | b. | In a 3 phase overhead system each line is suspended by a string of three insulators. The voltage at the top unit, near the tower and middle unit are 10kV and 11 kV respectively. i) Evaluate the ratio of shunt capacitance to self capacitance of each insulator. ii) String Efficiency iii) Line Voltage. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Deduce an expression for sag in overhead lines when i.Supports are at equal levels. ii. Supports are at unequal levels | CO2 | 10 |
|  | b. | A transmission line has a span of 150 m between level supports. The conductor has a cross-sectional area of 2 cm2 . The tension in the conductor is 2000 kg. If the specific gravity of the conductor material is 9.9 gm/cm3 and wind pressure is 1.5 kg/m length, calculate the sag. What is the vertical sag? |  | 10 |
| 7. | a. | Draw a cross sectional view, showing the constructional details of a single core low tension cable and explain the purpose of each material used. | CO2 | 10 |
|  | b. | Prove that gmax/gmin in a single core cable is equal to D/d where D is the internal sheath diameter and d is the core diameter. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A single core lead sheathed cable is graded by using three dielectrics of relative permittivity 5, 4 and 3 respectively. The conductor diameter is 2 cm and overall diameter is 8 cm. If the three dielectrics are worked at the same maximum stress of 40 KV / cm, find the safe working voltage of the cable. | CO2 | 15 |
|  | b. | Compare the merits and demerits of underground system versus overhead system. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | With a neat diagram explain feeders, distributors and service main of a systems. | CO3 | 5 |
|  | b. | At the conductor distributor AD is fed at A and D at 255 V and 250 V respectively and is loaded as shown. The resistance given are those of each conductor. Find the value of current in each section of the cable and the voltage at each load point. | CO3 | 15 |