Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : 14EE2013 |  | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : TRANSMISSION AND DISTRIBUTION |  | Max. Marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Show that transmitting high voltage reduces volume of the conductor, increases the transmission efficiency and decreases the % line drop with necessary equations. | CO1 | 10 |
|  | b. | A generating station has a maximum demand of 25MW, a load factor of 60%, a plant capacity factor of 50% and a plant use factor of 72%. Find i) the reserve capacity of the plant ii) the daily energy produced iii).maximum energy that could be produced daily if the plant while running as per schedule, were fully loaded. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Draw and explain the structure of AC transmission system. | CO1 | 10 |
|  | b. | Time; Consumer 1; Consumer 2; Consumer 3. 12 midnight to 8 A.M; No Load; 200W; No load. 8 A.M to 2 P.M; 600 W; No Load; 200 W. 2 P.M to 4 P.M; 200W; 1000 W; 1200 W. 4 P.M to 10 P.M; 800 W; No Load; No Load. 10 P.M to midnight; No Load; 200 W. The daily demands of three consumers are given below: Plot the load curve and find i) maximum demand of individual consumer ii) load factor of individual consumer iii) diversity factor iv) load factor of the station | CO1 | 10 |
| 3. | a. | Deduce an expression for line to neutral capacitance for a 3-phase overhead transmission line when the conductors are i) symmetrically placed ii) unsymmetrically placed but transposed. | CO2 | 15 |
|  | b. | Find the inductance per km of a 3-phase transmission line using 2·24 cm diameter conductors when these are placed at the corners of an equilateral triangle of each side 2 m. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | A 100-km long, 3-phase, 50-Hz transmission line has following line constants: Resistance/phase/km = 0·1 Ω ,Reactance/phase/km = 0·5 Ω. Susceptance/phase/km = 10 × 10−6 S. If the line supplies load of 20 MW at 0·9 p.f. lagging at 66 kV at the receiving end, calculate by nominal π method (i) sending end power factor (ii) regulation (iii) transmission efficiency. | CO2 | 10 |
|  | b. | Define ABCD constants as applied to the performance of medium transmission lines by nominal T method and explain. | CO2 | 10 |
| 5. | a. | In a three phase overhead system each line is suspended by a string of three insulators. The voltage at the top unit, near the tower and middle unit are 10kV and 11 kV respectively. i) Evaluate the ratio of shunt capacitance to self capacitance of each insulator. ii) String Efficiency iii) Line Voltage. | CO2 | 10 |
|  | b. | List out the factors affecting corono and discuss briefly about corono loss. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss the effect of wind and ice loading upon the mechanical design of transmission lines. | CO2 | 10 |
|  | b. | The towers of height 30 m and 90 m respectively support a transmission line conductor at water crossing. The horizontal distance between the towers is 500 m. If the tension in the conductor is 1600 kg, find the minimum clearance of the conductor and water and clearance mid-way between the supports. Weight of conductor is 1·5 kg/m. Bases of the towers can be considered to be at water level. | CO2 | 10 |
| 7. | a. | Show that the gmax/gmin in a single core cable is equal to D/d where D is the internal sheath diameter and d is the core diameter. | CO2 | 10 |
|  | b. | A single core cable has a conductor diameter of 1 cm and internal sheath diameter of 1.8 cm. If impregnated paper of relative permittivity 4 is used as the insulation, calculate the capacitance for 1 km length of the cable. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | With neat sketch explain the capacitance grading of cables and also derive an expression for the same. | CO2 | 10 |
|  | b. | A single-core 66 kV of conductor diameter 2 cm and lead sheath of diameter 5.3 cm is to be used on a 66 Kv, 3-phase system. Two intersheaths of diameter 3.1cm and 4.2 cm are introduced between the core and lead sheath. If the maximum stress in the layers is the same, find the voltages on the intersheaths. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe any three types of dc distributors and derive the expression for the same. | CO3 | 10 |
|  | b. | Derive an expression for AC distribution with respect to power factor referred to receiving end voltages. | CO3 | 10 |