Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER SYSTEM ANALYSIS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | # Illustrate in detail the need for system analysis in planning and operation of power system. | CO1 | 5 |
|  | b. | # Explain the modeling of synchronous generator, transformer, transmission line and static and dynamic loads of the power system. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. |  | # Consider the 50 Hz power system the single-line diagram of which is shown in Fig.The system contains three generators, three transformers and three transmission lines. The system ratings are Generator G1; 200 MVA, 20 kV, Xd = 15%. Generator G2; 300 MVA, 18 kV, Xd = 20%. Generator G3; 300 MVA, 20 kV, Xd = 20%. Transformer T 1; 300 MVA, 220Y/22 kV, Xd = 10%. Transformer T 2; Three single-phase units each rated 100 MVA, 130Y/25 kV, X = 10%. Transformer T 3; 300 MVA, 220/22 kV, X = 10%. # The transmission line reactances are as indicated in the figure.Draw the reactance diagram choosing the Generator 3 as the base. | CO1 | 20 |
| 3. | a. | Ybus of a power system is sparse. Infer the reason by considering a real power system. | CO1 | 5 |
|  | b. | Construct the ZBus for the sample system shown in the figure using Zbus building algorithm. If a new transmission line is installed between bus 2 and bus 3, how will you modify Zbus. . | CO1 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | In an unbalanced 3- system, the phase current Ia=10op.u, negative sequence current Ib2=150o p.u, zero sequence current Ico=390o p.u. Find the magnitude of phase current Ib in p.u. | CO1 | 5 |
|  | b. | Starting from basic conditions for a single line to ground fault with fault impedance Zf . Obtain the expression for fault current. Show the connection of sequence network for this fault. A 30 MVA, 11 kV generator has Z1=Z2= j0.15 pu and Zo=j0.025 pu. A line to ground fault occurs on the generator terminals. Find actual values of fault currents and line to line voltages during fault conditions. Assume that the generator neutral is solidly grounded and that the generator is operating at no load and at rated voltage at the occurrence of the fault. | CO2 | 15 |
| 5. | a. | Compare Gauss-Seidal, Newton-Raphson and Fast decoupled load flow studies describing their suitability, advantages, disadvantages and applications. | CO2 | 5 |
|  | b. | The following is the system data for a load flow problem. Calculate the voltage at the end of the second iteration using Gauss-Seidal method. Ybus=3−j9−2+j6−1+j30−2+j63.6667−j11−0.6667+j2−1+j3−1+j3−0.6667+j23.6667−j11−2+j60−1+j3−2+j63−j9 Bus code; Real Power Injection (P); Reactive Power Injection (Q); V. 1; -; -; 1.06. 2; 0.5; -0.2; -. 3; -0.4; -0.3; -. 4; -0.3; 0.1; -. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | Construct the Jacobian matrix for a three bus power system (one slack bus, one PV bus and one PQ bus ). Describe the mathematical expression for each element of the Jacobian matrix of the Newton-Raphson load flow studies. | CO2 | 5 |
|  | b. | Describe the Newton-Raphson load flow method with necessary mathematical equations and flow-chart. | CO2 | 15 |
| 7. | a. | Derive the coordination equations for ED considering transmission losses. | CO3 | 5 |
|  | b. | The fuel cost characteristics of four power plants are given by F1=0.0412P12+7.21P1+510 Rs./hr F2=0.0194P22+7.85P2+310 Rs../hr F3=0.0480P32+7.97P3+200 Rs./hr F4=0.0340P42+6.40P4+300 Rs./hr The power generation limits are Plant 1: 150≤ P1≤600 MW Plant 2: 100≤ P2≤500 MW Plant 3: 50≤ P3≤250 MW Plant 4: 100≤ P4≤350 MW Find the ED schedule and the total fuel cost for a demand of 1250MW. | CO3 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the various constraints in unit commitment problem with suitable examples. | CO3 | 10 |
|  | b. | Give the elementary idea of optimal load scheduling of Hydro - Thermal plants. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Define Power Quality Problem. If you want to evaluate the power quality of an electricity consumer, list down the steps you will follow. | CO2 | 5 |
|  | b. | From the fundamentals of rotational dynamics, derive the swing equation of a synchronous machine. How will you use the swing equation for stability analysis? | CO2 | 15 |