Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2024 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASICS OF ELECTRIC AND HYBRID VEHICLE | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the evolution of hybrid electric vehicles. | CO1 | 15 |
|  | b. | Compare and Contrast Electric vehicles and Gasoline vehicles | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | With a neat diagram, explain the working of 4 Stroke Spark Ignited IC engine. | CO1 | 15 |
|  | b. | If the brake power (Pb) is 25W and friction power (Pf) is 3W calculate the mechanical efficiency. | CO3 | 5 |
| 3. | a. | Portray the basic techniques for improving Engine Performance, Efficiency and Emissions. | CO1 | 15 |
|  | b. | Name the components of an electric drive train. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. |  | Illustrate the series hybrid, parallel hybrid and series-parallel configurations of hybrid vehicles in detail with neat diagrams. | CO2 | 20 |
| 5. | a. | Name the basic configurations for hybrid vehicles. | CO1 | 5 |
|  | b. | Explain the working of Solar Power Generation for Electric Vehicle. | CO2 | 10 |
|  | c. | List the applications of hybrid electric vehicle. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the working of Four Quadrant / Class E DC Chopper that can operate in all the four quadrants with neat diagrams. | CO3 | 15 |
|  | b. | A 4 pole, wave wound, DC shunt motor runs at 1000 rpm with a back emf of 200V, calculate the speed of the machine when its emf changes to 300V. | CO3 | 5 |
| 7. | a. | A DC to DC chopper operates from a 60V battery source in to a resistive load of 30 Ohms. The duty ratio is 0.6. Determine the average output voltage during step-up and step-down modes. | CO3 | 5 |
|  | b. | Compare and Contrast brushed DC motor with brushless DC motor. | CO1 | 5 |
|  | c. | Elucidate the working of VSI fed three phase induction motor with neat diagram. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the working of any one type of power converter used for Switched Reluctance Motor with necessary diagram. | CO1 | 15 |
|  | b. | Outline the basic SRM drive system. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the constructional features, advantages and disadvantages of Lithium Ion battery. | CO2 | 10 |
|  | b. | Brief out Super Capacitors with neat diagram | CO3 | 10 |