Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EE2027 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HVDC AND FACTS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. |  | Summarize the operating problems have surfaced that affected reliable operation and in few cases even lead to catastrophic failures of HVDC Technology. | CO1 | 10 |
|  | b. |  | Describe with three types of DC link configuration and a schematic diagram of a typical HVDC converter station. | CO1 | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. |  | Explain the VSC based HVDC Transmission system with its advantages. | CO1 | 10 |
|  | b. |  | Discuss in detail the relative merits of the two modes of transmission which need to be considered by a system planner are based on economics of transmission, technical performance and reliability. | CO2 | 10 |
| 3. | a. |  | Describe i) Firing angle control, ii) Individual Phase Control and Equidistant Pulse Control. | CO2 | 10 |
|  | b. |  | Discuss in detail the HVDC converter control characteristics, negative current margin and modified characteristics including VDCOL. | CO2 | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. |  | Describe the principles of DC Link control with a schematic and steady state equivalent circuit. | CO2 | 10 |
|  | b. |  | A bipolar HVDC Link operates with + 500 kV, 1000 MW [rated voltage and power] at the rectifier end. The resistance of each line conductor is 20 ohms. The converter transformers have a leakage reactance of 0.15 p.u on their own bases. There are two series connected converter bridges per pole. The AC bus voltage at both rectifier and inverter buses are regulated at 400 kV rated. [a]. Compute the ratings and turns ratios of the converter transformers at the rectifier and inverter [b]. Find the reactive power supplied at the rectifier and inverter stations, assume operating values of αr = γi = 12° where αr , γi is the delay angle, extinction angle of rectifier, inverter | CO3 | 10 |
| 5. | a. |  | Enumerate the detailed classification of FACTS controllers for Transmission and Distribution system. | CO1 | 10 |
|  | b. |  | Show the General equivalent circuit with constraint equations and control variables of a FACTS controller. | CO1 | 10 |
| (OR) |  |  |  |  |  |
| 6. | a. |  | Discuss with schematic diagram of VSC based FACTS controllers. | CO1 | 10 |
|  | b. |  | List out the benefits of FACTS controller and applications in distribution system. | CO1 | 10 |
| 7. | a. |  | Explain about A Typical SVC (TSC-TCR) Configuration. | CO1 | 10 |
|  | b. |  | Analyze A transmission line with SVC connected at midpoint with its Control characteristic. | CO1 | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. |  | Write down the Simplified Analysis of a Three Phase Six Pulse STATCOM. | CO2 | 10 |
|  | b. |  | Describe the various operating modes with Single Line Diagram of a TCSC. | CO2 | 10 |
|  |  |  | Compulsory: |  |  |
| 9. | a. |  | Explain the operation of UPFC Connected at the Sending End. | CO3 | 10 |
|  | b. |  | The Unified Power Flow Controller (UPFC) proposed by Gyugyi is the most versatile FACTS controller – Discuss in detail. | CO3 | 10 |