Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2031 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # RENEWABLE ENERGY-I | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Briefly explain the mismatch losses of solar PV system. | CO1 | 10 |
|  | b. | Draw and discuss the effect of temperature and radiation on the output of solar cell. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Derive the output voltage equation of a solar cell using the single diode equivalent circuit. | CO1 | 15 |
|  | b. | Mention the different electrical parameters mentioned in the datasheet of solar panel | CO1 | 5 |
| 3. | a. | Make an inverter for a PV system with the output of 230V and 50 Hz and explain how the output can be controlled using sinusoidal pulse with modulation for less THD | CO1 | 15 |
|  | b. | Draw the output characteristic of the solar cell to indicate the MPP. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Track the maximum power point of SAPV system using P&O algorithm and mention the drawback of the same algorithm. | CO2 | 15 |
|  | b. | Mention the role of charge controller in the SAPV system. | CO2 | 5 |
| 5. | a. | Discuss the operation of shunt type and series type charge controllers. | CO2 | 15 |
|  | b. | Discuss the different types of batteries used in SAPV system and their limitations. | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Analyse the maintainence of MPP voltage using buck-boost DC-DC Converter in SAPV system. | CO2 | 15 |
|  | b. | Draw and explain the operation of SAPV system with MPPT. | CO2 | 5 |
| 7. |  | Design a complete SAPV system for supplying the load of your home. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Draw and explain the operation of horizontal axis wind turbine. | CO3 | 12 |
|  | b. | Derive the equation of output power generated from the kinetic energy of the wind turbine. | CO3 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the operation of vertical axis wind turbine. | CO3 | 10 |
|  | b. | Draw and Explain the operation of grid connected wind energy system. | CO2 | 10 |