Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EE2035 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SWITCHED MODE POWER SUPPLIES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Compare the characteristics of a Linear regulator with Switched Mode regulator. | CO1 | 14 |
|  | b. | Why are Switched Mode regulator so much more efficient than Linear regulator? | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. |  | Describe the operation of a Forward converter. What is the need of a teritary winding in a practical forward converter? | CO1 | 20 |
| 3. |  | Explain the working of a Flyback converter both in continuous and discontinuous mode. Derive the voltage conversion ratio, current through the winding of the transformer and voltage ripple in both the modes. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Design a full-bridge converter with the following specification: Vo = 5V, Po(max) = 50W, Io(max) = 10A, Io(min) = 1A, Vdc(max) = 60V, Vdc(min) = 38V, switching frequency = 50kHz. | CO1 | 20 |
| 5. | a. | What does Current Mode control mean for a switching converter? Compare the properties of Voltage fed regulator topologies with current fed regulator topologies. | CO2 | 14 |
|  | b. | Enumerate the limitations of Current Mode control. | CO2 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the steps of a designing a transformer for a push pull converter based on core and bobbin area (Ae,Ab), current density, frequency and peak flux density. | CO3 | 14 |
|  | b. | Write the characteristics of different types of transformer core. | CO3 | 6 |
| 7. |  | Design a choke for a buck regulator with the following specifications: Input Voltage = 25V, Output Voltage = 5V, Maximum Output Current = 10A, Frequency = 25kHz, Maximum Ripple Current = 20% of Output Current, Maximum Temperature Rise = 30◦C above ambient. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Design the RCDsnubber circuit for the forward converter with the peak current Ip just before turn “off” was 3.45 A, transistorfall time was 0.3 μs, and transistor dissipation without a snubberwas 19W. | CO3 | 10 |
|  | b. | Write short notes on the switching characteristics of IGBT. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the working of a resonant forward converter both in continuous and discontinuous operating modes. | CO2 | 20 |