Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EI2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SENSORS AND TRANSDUCERS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | What are the functional elements of a measurement system? Explain the function of each element with a suitable example. | CO1 | 10 |
|  | b. | A diaphragm type pressure measuring instrument is calibrated for absolute pressures of 6 to 760 mm of Mercury. It has an accuracy of ± 1%. Calculate the scale range, scale span and maximum static error. | CO1 | 5 |
|  | c. | What is static calibration? Mention the significance of the static calibration curve in measuring instruments. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Classify the types of instruments used in measurement applications. | CO1 | 15 |
|  | b. | A moving coil voltmeter bas a uniform scale with 100 divisions The full scale reading is 200 V and 1/10 th of a scale division can be estimated with a fair degree of certainity. Determine the resolution of the instrument in volt. | CO1 | 5 |
| 3. | a. | List the types of errors encountered in measurement systems. Illustrate with examples how the errors are handled. | CO1 | 10 |
|  | b. | Ten measurements of a pressure made by an instrument at different time intervals give the following readings in kg/cm2 : 7.92, 8.03, 8.10, 7.95, 8.01, 7.98, 7.95, 8.07, 7.94 and 7.97. Find arithmetic mean, standard deviation, and most probable error. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Mention the factors that influence the choice of a transducer. | CO2 | 10 |
|  | b. | A thermometer is calibrated from 100C to 1100C. The accuracy is specified within 0.1% of instrument span. What is the maximum static error? | CO1 | 5 |
|  | c. | Differentiate (i) Active and Passive Transducers (ii) Analog and Digital Transducers. | CO2 | 5 |
| 5. | a. | # Show how a simple potential divider circuit can be used for displacement measurement. Classify the types of resistive potentiometers and discuss about its resolution and sensitivity. | CO2 | 12 |
|  | b. | A semiconductor gauge having a resistance of 1000 ohms and a gauge factor of 133 is subjected to a compressive strain of 500 microstrain. Calculate the new value of resistance of the gauge. | CO2 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | A Platinum resistance thermometer has a resistance of 100ohmsat 25°C. Find its resistance at 65°C. The resistance temperature co-efficient of Platinum is 0.00392 /0C. | CO2 | 5 |
|  | b. | State Seebeck Effect. Demonstrate how it is used for temperature measurement with relevant diagrams and explanations. | CO2 | 15 |
| 7. | a. | Draw the input–output characteristic of LVDT. Explain the phase sensitive demodulation circuits and null–reduction technique used for LVDT. | CO2 | 10 |
|  | b. | A capacitive transduceruses two quartz diaphragms of area 750 mm2 separated by a distance of 3.5 mm. A pressure of 900 kN/m2 when applied the top diaphragmproduces a deflection of 0 6 mm. The capacitance is 370 pF when no pressure is applied to thediaphragms. Find the value of capacitance after the application of a pressure of 900 kN/m2 | CO3 | 10 |
| (OR |  |  |  |  |
| 8. | a. | With neat diagrams, explain the principle of operation of synchros. Illustrate how it can be used for angle telemetry. | CO2 | 10 |
|  | b. | With neat sketch, explain the working of capacitive microphone. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Define magnetostrictive effect. Name the materials used in magnetostrictive transducers. Describe any one application of magnetostrictive transducer. | CO2 | 10 |
|  | b. | Illustrate how an optical shaft encoder can be used to measure angular position. | CO3 | 10 |