Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EI2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CONTROL SYSTEM | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Determine the transfer function Y2(S)/F(S) of the system shown in fig. f(t) | CO1 | 12 |
|  | b. | Explain open and closed loop system with one example. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Find transfer function for the following Signal flow graph. | CO1 | 8 |
|  | b | Resolve the transfer function Y(S)/R(S) of the system shown in fig. |  | 12 |
| 3. | a. | Derive the response of underdamped second order system for unit step input. | CO2 | 15 |
|  | b. | Find static error Kp, Kv and Ka for the following unity feed back system whose open loop transfer function Gs=20S+2ss+1s+3 | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Define rise time,delay time, peak time and settling time | CO2 | 5 |
|  | b. | The open loop transfer function of a unity feedback system is given by Gs=Kss+10. Determine the gain K by comparing closed loop transfer function with standard transfer function. So that the system will have a damping ratio of 0.5. For this value of K, determine settling time, peak overshoot and time at peak overshoot for a unit step input. | CO2 | 15 |
| 5. | a. | What do you infer from the number of sign changes of the terms in the first column of the Routh array? | CO3 | 5 |
|  | b. | Given the system below find the range of the gain K that will lead to stability, unstability and marginal stability. | CO3 | 15 |
| (OR) |  |  |  |  |
| 6. |  | A unity feedback system has an open loop transfer function Gs=KSS2+4S+13 Sketch the root locus. | CO3 | 20 |
| 7. | a. | Check the stability of the system using Routh Hurwitz Criteria whose characteristic equation is given by S5+S4+2S3+2S2+3S+15=0 | CO3 | 13 |
|  | b. | Determine the stability of the following unity feed back system using Routh Hurwitz Criteria. Gs=9S2s+2 | CO3 | 7 |
| (OR) |  |  |  |  |
| 8. |  | Sketch the root locus of the system, whose open loop transfer function is Gs=Kss+2s+4 | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Sketch the Bode plot for the following transfer function and determine the phase and gain cross over frequencies. Gs=10s1+0.4s1+0.1s | CO2 | 15 |
|  | b. | List out all frequency domain specifications. Define gain margin and phase margin. | CO2 | 5 |