Reg.No. ____________

End Semester Examination – April/May – 2018

| # Code : | # 14EI2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRONIC INSTRUMENTATION | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define the following terms as applied to the measuring instrument: - Range - Accuracy - Sensitivity - Linearity - True value | CO1 | 10 |
|  | b. | Sketch a shunt ohmmeter circuit as used with an electronic voltmeter. Explain the circuit operation and compare it to a series ohmmeter. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Describe with an examples how the following measurement can be made with the use of CRO: - Voltage - Frequency - Phase angle | CO1 | 10 |
|  | b. | The half-wave electronic voltmeter uses a 500µA deflection meter with a 460Ω coil resistance. If Rs=450Ω calculate the VRMS required to give FSD. | CO3 | 10 |
| 3. | a. | Exemplify the block diagram of Q meter. Explain its operation. | CO2 | 10 |
|  | b. | A circuit consisting of an unknown coil, a resistance and a variable capacitor connected in series is tuned to resonance using a Q meter. If the frequency is 450kHz and the resonating capacitor is set at 250pF, determine the effective inductance and resistance of unknown coil. The Q meter indicates 105 and the resistance is 0.75 Ω. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Draw the circuit diagram to show how current can be measured using an op-amp electronic voltmeter. Explain the burden voltage. | CO3 | 10 |
|  | b. | Build the block diagram of Vector Impedance meter and explain its operation in detail. | CO2 | 10 |
| 5. | a. | A 20V DC voltage is measured by an analog and digital multimeter. The analog instrument is on its 25V range and its specified accuracy is ±2%. The digital meter has a 31/2 digit display and an accuracy of ±(0.6 + 1). Determine the measurement accuracy in each case. | CO1 | 10 |
|  | b. | Sketch the block diagram and system waveforms for a DVM using a ramp generator type ADC. Explain its operation. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Draw the front panels of a typical digital multimeter, showing the terminals and controls. Explain terminal connections, function selection, range selection, and meter readings. | CO3 | 10 |
|  | b. | Draw the basic block diagram of a digital frequency meter, sketch the system waveforms and explain the instrument operation. | CO3 | 10 |
| 7. | a. | Sketch the construction of an LVDT and its circuit diagram. Explain LVDT operation. | CO3 | 10 |
|  | b. | Draw an op-amp monostable multivibrator circuit. Show the waveforms at various points in the circuit and explain its operation. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Sketch a basic block diagram of an RF signal generator. Identify each component of the diagram, and briefly explain. | CO3 | 10 |
|  | b. | A 1kHz triangular wave with a peak amplitude of 10V is applied to the vertical deflection plates of a CRT and a 1kHz sawtooth wave with a peak amplitude of 20V is applied to the horizontal deflection plates. The CRT has a vertical deflection sensitivity of 0.4cm/V, and a horizontal deflection sensitivity of 0.25 cm/V. Assuming that the two inputs are synchronized, determine the waveform displayed on the screen. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Compare virtual instruments with traditional instruments. | CO2 | 10 |
|  | b. | Explain in detail about the process involved in LabVIEW environment. | CO2 | 10 |