Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EI2013 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | INDUSTRIAL DATA COMMUNICATION NETWORKS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Describe in detail with necessary diagrams, how UART functions as a transmitter and receiver? | CO1 | 15 |
|  | b. | Suggest and justify a high speed UART for high speed communication. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | What is the need for industrial standards? List few industrial standards. | CO1 | 5 |
|  | b. | For the digital data 11010111 represent the signal pattern using different line coding methods | CO1 | 15 |
| 3. | a. | How long will it take to transfer a 1.2-MB file using a modem link working at 52,000 bits/sec? | CO1 | 4 |
|  | b. | Explain in detail about the most commonly used parallel interface standard IEEE 488. | CO1 | 16 |
| (OR) |  |  |  |  |
| 4. | a. | RS 485 preffered in Industries for multipoint interface. Justify your answer. | CO2 | 5 |
|  | b. | Explain the architecture of USB with the reference to OSI layers. | CO2 | 15 |
| 5. |  | Discuss in detail about the SDLC and HDLC protocol. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | The MAC sub-layer is a part of Network layer. Justify | CO2 | 4 |
|  | b. | Discuss in detail about the different Medium Access Control methods. | CO2 | 16 |
| 7. | a. | What are the different modes in Modbus communication | CO3 | 4 |
|  | b. | Discuss in detail about the modbus protocol. | CO3 | 16 |
| (OR) |  |  |  |  |
| 8. | a. | Draw the response frame of MODBUS protocol for reading the status of the coil with address 000B and content FFFF. | CO3 | 4 |
|  | b. | Discuss in detail about the IEEE802.3 LAN standard. | CO3 | 16 |
|  |  | Compulsory: |  |  |
| 9. | a. | List the functions of Network support and User support layers of OSI model | CO3 | 15 |
|  | b. | Justify the need of physical address and logical address | CO3 | 5 |