Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EI2024 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | POWER PLANT INSTRUMENTATION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss on the selection of site and the processes involved in Nuclear Power Generation. | CO1 | 16 |
|  | b. | List the objectives of instrumentation in power plants. | CO2 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the power generation techniques in thermal power plant. | CO1 | 14 |
|  | b. | Illustrate how chemical energy of fuel is converted into electrical energy. | CO1 | 6 |
| 3. | a. | Show the significance of condensation pot in flow measurement system with suitable diagram. | CO2 | 10 |
|  | b. | Represent the steam flow control using P&I diagram. | CO2 | 6 |
|  | c. | Identify how pressure is measured in hot water pipe lines? | CO2 | 4 |
| (OR) |  |  |  |  |
| 4. | a. | Analyse the operation of control rooms present in the thermal power plant with a neat sketch. | CO2 | 14 |
|  | b. | State the merits of solar power generating system based on cost, safety, and pollution. | CO1 | 6 |
| 5. | a. | Examine the water flow measurement in the following configuration. i) DPT below pipeline ii) DPT above the pipeline | CO2 | 10 |
|  | b. | The raw water pressure is to be measured with a pressure transmitter which is located 20m below the waterline. What is the absolute error and percentage error if the waterline pressure is 4.5 kg/cm2 and transmitter is calibrated for 0-5 kg/cm2 | CO2 | 6 |
|  | c. | Interpret the Deaeration process in feed water treatment. | CO2 | 4 |
| (OR) |  |  |  |  |
| 6. | a. | The orifice plate sensor that is used to measure the feed water flow in the boiler is designed to give the maximum differential pressure of 10m for 200 T/hr water flow. The differential pressure transmitter is used for converting the differential pressure to a 4-20mA dc linear signal. A square root extractor circuit is introduced to convert the current signal to a voltage signal (1-5 V) linear to flow rate. Design measurement system for the above problem and give the calibration chart for calibrating the transmitter along with square root extractor. | CO2 | 14 |
|  | b. | Design a current to voltage converter used in process industries. | CO2 | 6 |
| 7. | a. | Explain the measurement system for measuring the content of CO and CO2 present in flue gas. | CO2 | 10 |
|  | b. | Suggest a suitable instrument for conductivity measurement of feed water and explain its working. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the methodologies adopted in water side steam temperature control. | CO3 | 10 |
|  | b. | Describe the series and parallel air fuel ratio control. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Examine the application of DCS in thermal power plant. | CO3 | 12 |
|  | b. | Describe the interlocking operations in a boiler for safety purposes. | CO3 | 8 |