Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EI2046 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PROCESS CONTROL FOR FOOD ENGINEERS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the Laplace Transform of cos(at). | CO1 | 10 |
|  | b. | Write short note on servomechanism with neat sketch. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the design concepts involved in PID controller with an suitable example. | CO3 | 10 |
|  | b. | Explain in detail about the Construction and working of pneumatically operated Control valve with neat sketch. | CO3 | 10 |
| 3. | a. | Use Mason’s gain formula to determine the overall transfer function of the system shown in Figure. | CO1 | 15 |
|  | b. | Give any five rules of Block Diagram Reduction. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Determine the overall transfer function C1/R1 for the system shown in figure using Block Diagram reduction rules. | CO1 | 15 |
|  | b. | With the neat diagram explain about Gas Chromatography. | CO3 | 5 |
| 5. | a. | Compare RTD & Thermistor transducers and Illustrate your answer by defining the output from each, stating approximate temperature ranges that can be measured and sketch the relationship between temperature and output for each transducer. Also, list the applications of temperature transducer in food industry. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Determine the stability of the system using Routh array method whose characteristics equation given by | CO2 | 15 |
|  | b. | Mention the use of two switch controller in food making instruments. | CO3 | 5 |
| 7. | a. | Briefly explain about the principle of capacitive pressure transducer with a neat diagram. | CO3 | 10 |
|  | b. | List the Non-contact type of level transducers and Explain with the neat sketch, the construction and working principle. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the concept in measuring Density using Air bubbler system and photoelectric transducer. | CO3 | 14 |
|  | b. | Write the principle in measuring the level using Capacitance. | CO3 | 6 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain in detail about the construction and working of Conductivity meter. | CO3 | 5 |
|  | b. | Explain in detail about the construction and working of pH meter. | CO3 | 15 |