Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EI2048 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INSTRUMENTATION AND CONTROL SYSTEMS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw the block diagram representation of a generalized measuring system, identify the various elements and point out the function performed by each element. | CO1 | 15 |
|  | b. | How galvanometer can be converted into an ammeter? | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Describe briefly the construction and working of strip chart recorder. | CO1 | 15 |
|  | b. | Define error and list the types of error. | CO1 | 5 |
| 3. | a. | Discuss the constructional features of Bourdon-tube pressure gauge. | CO1 | 15 |
|  | b. | Describe the working principle of energy meter. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Explain briefly the construction and working of a RTD for measurement of Temperature. | CO1 | 15 |
|  | b. | Explain how the Wheatstone bridge circuit may be utilized for the measurement of temperature. | CO1 | 5 |
| 5. | a. | Explain the working of a hot wire anemometer for constant current method. | CO1 | 15 |
|  | b. | Explain the working of Electromagnetic flow meter. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | With relevant diagram explain the different types of strain gauge. | CO1 | 15 |
|  | b. | Explain gauge factor and list its significance. | CO1 | 5 |
| 7. | a. | Discuss in detail the open loop and closed loop control system with suitable example? | CO1 | 15 |
|  | b. | Describe the necessary condition for stability. Explain the relation between stability and coefficient of characteristic polynomial. | CO1 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Find the overall transfer function of the system for the signal flow graph shown below. | CO3 | 15 |
|  | b. | Describe the basic properties of signal flow graph. | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Construct Routh array and determine the stability of the system represented by the characteristic equation, s6+2s5+8s4+12s3+20s2+16s+16=0. Comment on the location of the roots of characteristic equation. | CO2 | 15 |
|  | b. | Find the type and order of the following system transfer function (i) G(s)=K(s+1)(s+2) (ii) G(s)=ks(s+1) | CO2 | 5 |