Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HEAT AND MASS TRANSFER | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | It is proposed to build a cold store having an outer wall of concrete (100 mm thick) and an inner wall of wood (10 mm thick), with the space in between (100mm) ﬁlled with polyurethane foam. If the inner wall temperature is 5C and theouter wall is maintained at the ambient air temperature of 20◦C, - Calculate the rate of heat penetration, - Calculate the temperature at the interfaces of the three layers.K of polyureathane is 2.6 X 10 -4 W/cm °C | CO2 | 10 |
|  | b. | In an experiment to measure the thermal conductivity of meat, beef was formed into a squaresection block 6 cm × 6 cm and 1 cm thick. The edges of the block were insulated and heat was supplied continuously to one face of the block at a rate of 0.80 W. The temperatures of each face were measured with thermocouples and found to be 28.5 and 23.3◦C, respectively. What is the thermal conductivity of beef? | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Derive an expression for heat transfer in conduction through multilayered rectangular walls in series. | CO2 | 10 |
|  | b. | A stainless-steel pipe (thermal conductivity 15 W/m K) is being used to transport heated oil at 125°C. The inside temperature of the pipe is 120°C. The pipe has an inside diameter of 5 cm and is 1 cm thick. Insulation is necessary to keep the heat loss from the oil below 25 W/m length of the pipe. Due to space limitations, only 5 cm thick insulation can be provided. The outside surface temperature of the insulation must be above 20°C (the dew point temperature of surrounding air) to avoid condensation of water on the surface of insulation. Calculate the thermal conductivity of insulation that will result in minimum heat loss while avoiding water condensation on its surface. | CO3 | 10 |
| 3. | a. | Sucrose syrup flows in a pipe of 0.023 m inside diameter at a rate of 40 lt/min, while steam is condensing on the outside surface of the pipe. The syrup is heated from 50 to 70 °C, while the inside wall temperature is at 80 °C. Calculate i) the heat transfer coefficient and ii) the required length of the pipe. Take 60=1200kg/m3, 60=3.8 cP, 80=3.8 cP, cp60=3120 J/kg℃, kp=0.46 W/m℃ | CO2 | 10 |
|  | b. | Describe the applications of dimensionless numbers in estimation of heat transfer coefficient. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. |  | In a fruit packaging house, oranges are washed, dried in a stream of high speed air at room temperature, waxed, and dried again in a hot air stream. Calculate the heat transfer coefficient on the surface of an orange if the air velocity is 10 m/s, the air temperature is 55 °C, the orange surface temperature is 25 °C, and the orange has a spherical shape with a diameter of 8 cm. | CO2 | 20 |
| 5. | a. | In a canning process, 100 cans per minute are to be heated from 60 °C to 100 °C, and demoistured of the adhering moisture to the cans. Each can contains on average 0.3 m1 of water adhering to it. The cans have an emissivity of 0.7 and an average area of each can is 0.047 m2, and its weight is 100 g. The specific heat of can material is 0.25 kJ/kg°C. Calculate the temperature of the oven considering the radiative heat transfer only. | CO2 | 10 |
|  | b. | Consider a 20 cm diameter spherical ball at 800 K suspended in air. Assuming the ball closely approximates a blackbody, determine i) the total blackbody emissive power, ii) the total amount of radiation emitted by the ball in 5 min. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | A continuous length of biscuit 0.20 m wide is baked in an oven which consists of a conveyor belt in a square section enclosure, the roof of which contains a radiative element 0.20 m wide. The biscuit is baked at 180°C and is conveyed directly beneath the element with a vertical separation of 0.40 m. The emissivities of element and biscuit are 0.95 and 0.80, respectively. If the rate of heat exchange is 900 W m− 1 , what will be the temperature of the element? | CO3 | 10 |
|  | b. | Glass bottles may be prevented from breaking on ﬁlling with hot pasteurized juice when their temperature is close to that of the juice being ﬁlled. The bottles are rapidly heated by passing through a chamber that has top, bottom, and side walls heated by natural gas. The glass bottles may be considered as an object completely surrounded by a radiating surface. The glass bottles have an emissivity of 0.94, a mass of 155 g each, a speciﬁc heat of 1256 J/(kg.K), and a surface area of 0.0219 m2 . If the glass bottles are to be heated from 15.5°C to 51.6°C in 1 minute, calculate the temperature of the walls of the chamber to achieve this average heating rate when the glass is at the midpoint of the temperature range (33.6°C). | CO2 | 10 |
| 7. | a. | Illustrate with a neat diagram working 1-2 Heat exchanger. | CO2 | 10 |
|  | b. | Describe with a neat sketch the construction and working of scraped surface heat exchanger. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. |  | A counter-flow double-pipe heat exchanger is to heat water from 20°C to 80°C at a rate of 1.2 kg/s. The heating is to be accomplished by geothermal water available at 160°C at a mass flow rate of 2 kg/s. The inner tube is thin-walled and has a diameter of 1.5 cm. If the overall heat transfer coefficient of the heat exchanger is 640 W/m2 °C, determine the length of the heat exchanger required to achieve the desired heating. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe with a neat sketch where ever applicable the convective theory of mass transfer. | CO2 | 15 |
|  | b. | Write a note on eddy diffusion. | CO3 | 5 |