Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HEAT AND MASS TRANSFER | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive an expression for heat transfer in conduction through hollow cylinder. | CO1 | 8 |
|  | b. | A furnace is made out of a steel wall followed by a brick wall followed by a sil-o-cel brick wall. The inside temperature of the furnace is 350°C and the outside temp is 40°C. Find the rate of heat loss per m2 and temperatures at the junction of the steel and brick wall and at the junction of the brick and sil-o-cel brick walls. Properties of the materials are as follows: | CO3 | 10 |
|  | c. | State Fourier law of heat conduction. | CO1 | 2 |
| (OR) |  |  |  |  |
| 2. | a. | A 3-mm-diameter and 5-m-long electric wire is tightly wrapped with a 2-mm thick plastic cover whose thermal conductivity is k = 0.15 W/m°C. Electrical measurements indicate that a current of 10A passes through the wire and there is a voltage drop of 8 V along the wire. If the insulated wire is exposed to a medium at T = 30°C with a heat transfer coefficient of h = 12 W/m2 °C, determine the temperature at the interface of the wire and the plastic cover in steady operation. Also determine whether doubling the thickness of the plastic cover will increase or decrease this interface temperature. | CO3 | 10 |
|  | b. | A stainless-steel pipe (thermal conductivity 17 W/[m °C]) is being used to convey heated oil. The inside surface temperature is 130 °C. The pipe is 2 cm thick with an inside diameter of 8 cm. The pipe is insulated with 0.04 m thick insulation (thermal conductivity 0.035 W/m°C). The outer insulation temperature is 25°C. Calculate the temperature of the interface between steel and insulation, assume steady-state conditions. | CO3 | 10 |
| 3. | a. | Enumerate the steps for calculation of convective heat transfer coefficients using empirical correlations. | CO2 | 10 |
|  | b. | Water flows in a pipe of 0.0475m inside diameter at a velocity of 1.5m/s. Calculate the heat transfer coefficient if the temperature of the water is 60°C and 40°C at the inlet and the outlet of the pipe respectively, and the inside wall temperature of the pipe is 35 °C. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Write in detail about convection heat transfer. | CO3 | 10 |
|  | b. | Water flowing at a rate of 0.02 kg/s is heated from 20 to 60 °C in a horizontal pipe (inside diameter 5 2.5 cm). The inside pipe surface temperature is 90°C. Estimate the convective heat-transfer coefficient if the pipe is 1m long. | CO2 | 10 |
| 5. | a. | A loaf of bread is passing through a baking oven, the walls of which are maintained at a constant temperature of 220°C. The bread has an area of 0.09 m2 and is at 100 °C. The emissivity of bread may be taken as 0.52. In addition to radiation heat, there is convective heat also by air at 220 °C. Calculate the heat transfer rate. | CO2 | 10 |
|  | b. | State the following laws of radiation: i. Stefan-Boltzmann law ii. Plank’s law iii. Wien’s displacement law iv. Kirchoff’s law | CO2 | 8 |
|  | c. | Differentiate thermal radiation from conduction and convection. | CO1 | 2 |
| (OR) |  |  |  |  |
| 6. | a. | In a canning process, 100 cans per minute are to be heated from 60 °C to 100 °C, and demoistured of the adhering moisture to the cans. Each can contains on average 0.3 m1 of water adhering to it. The cans have an emissivity of 0.7 and an average area of each can is 0.047 m2 , and its weight is 100 g. The specific heat of can material is 0.25 kJ/kg°C. Calculate the temperature of the oven considering the radiative heat transfer only. | CO2 | 15 |
|  | b. | Write a note on Black and Grey bodies. | CO1 | 5 |
| 7. |  | A juice with cp value 4.0 kJ/kg °C, flows in the inner pipe of a tubular heat exchanger. The juice enters the heat exchanger at 20°C and exits at 60°C .The flow rate of juice is 0.5 kg/s. In the annular section, hot water at 90°C enters the heat exchanger and flows counter currently at a flow rate of 1 kg/s. The average specific heat of water is 4.18 kJ/(kg°C). Assume steady-state conditions. - Calculate the exit temperature of water. - Calculate log-mean temperature difference. - If the average overall heat transfer coefficient is 2000 W/(m2 °C) and the diameter of the inner pipe is 5 cm, calculate the length of the heat exchanger. - Repeat these calculations for parallel flow configuration | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Steam in the condenser of a power plant is to be condensed at a temperature of 30°C with cooling water from a nearby lake, which enters the tubes of the condenser at 14°C and leaves at 22°C. The surface area of the tubes is 45 m2 , and the overall heat transfer coefficient is 2100 W/m2 · °C. Determine the mass flowrate of the cooling water needed and the rate of condensation of the steam in the condenser. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Give in detail about the theory and principle behind diffusion. | CO2 | 15 |
|  | b. | Write down the role of diffusion in mass transfer. | CO3 | 5 |