Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # HEAT AND MASS TRANSFER | # Max. marks : | # 100 |

(for Biotech Only)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the expression for heat transfer through furnace wall made of three different materials in series. Assume k1, k2 and k3 be the thermal conductivities of materials and x1, x2 and x3 be the respective thickness. Assume hot face and cold face temperatures be T1 and T2 respectively. | CO2 | 10 |
|  | b. | A 50mm diameter pipe of circular cross section and with walls 3mm thick is covered with two concentric layers of logging. The inner layer having a thickness of 25mm and thermal conductivity of 0.08 (W/m K) and outer layer having thickness of 40mm and thermal conductivity of 0.04 (W/m K) .Estimate the rate of heat loss per meter length of pipe and the temperature inside the pipe is 500K and outside surface temperature is 330K for pipe and thermal conductivity is 45W/m K. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A furnace is constructed with 225mm thick of fire brick, 120mm of insulating brick and 225mm of the building brick. The inside temperature is 1200K and the outside temperature is 330K.Find the heat loss per unit area and the temperature of the junction of the fire brick and insulation brick. Data:Thermal conductivity of fire brick= 1.4 (W/m K), Thermal conductivity of insulating brick= 0.2 (W/m K) and Thermal conductivity of building brick= 0.7 (W/m K). | CO3 | 10 |
|  | b. | Derive and express the heat transfer through the hollow cylinder with neat sketch. | CO3 | 5 |
|  | c. | Derive the heat conduction through a composite wall. | CO3 | 5 |
| 3. |  | Calculate the heat transfer coefficient for fluid flowing through a tube having inside diameter 40mm at a rate of 5500kg/h. Assume that fluid is being heated. Data: Properties of fluid at mean bulk temperature: Viscosity of flowing fluid = 0.004 N.s/m2 Density of flowing fluid = 1.07 g/cc Specific heat of flowing fluid = 2.72 kJ/kg.K Thermal conductivity of flowing fluid = 0.256 W/m.K | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | State and explain the various laws of black body radiation? | CO3 | 5 |
|  | b. | Calculate the rate of heat loss from a 6m long horizontal steam pipe, 60mm OD when carry steam at 800 kN/m2. The temperature of atmosphere and surroundings is 290K. Data: emissivity, e = 0.85 and Stefan-Boltzmann constant σ = 5.67 x 10-8 W/m2.K4 Film coefficient (hc) for heat loss by natural convection can be calculated by: hc = 1.64 ΔT 0.25 W/m2K steam is saturated at 800 kN/m2 and 443K | CO1 | 15 |
| 5. |  | It is desired to warm 0.90 kg/s of air flowing through tubes from 283K to 366K with the help of flue gas entering at 700K and leaving at 366K in counter current fashion. Calculate the heat transfer area of heat exchange equipment. Data: Pipe diameter = 12mm Mass flow velocity (G) = 19.9 kg/m2 s Thermal conductivity of air = 0.029 W/mK Viscosity of air = 1.98 x10-5 N.s/m2 Specific heat of air = 1 kJ/kg K ho = 232 W/m2 K | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Describe the shell and tube heat exchanger with neat sketch. | CO1 | 10 |
|  | b. | Water enter a two fluid in heat exchanger at 55ºC and leaves at 85ºC.Hot gases enter at 305ºC and leaves at 160ºC. If the total heat transfer area is 500 square meter and the overall heat transfer coefficient is 600 kcal/hrm2ºC. Determine the total heat transferred per hour for (i) Parallel flow, (ii) counter flow. | CO1 | 10 |
| 7. | a. | A liquid food (specific heat4.0 kJ/kg°C) flows in the inner pipe of a double pipe heat exchanger. The liquid food enters the heat exchanger at 20°Cand exits at 60°C .The flow rate of the liquid food is 0.5 kg/s. In the annular section, hot water at 90°C enters the heat exchanger and flows counter currently at a flow rate of 1 kg/s.The average specific heat of water is 4.18 kJ/(kg°C). Assume steady- state conditions. - Calculate the exit temperature of water. - Calculate log-mean temperature difference. - If the average overall heat transfer coefficient is 2000W/(m2°C) and the diameter of the inner pipe is 5 cm, calculate the length of the heat exchanger - Repeat these calculations for parallel flow configuration. | CO1 | 15 |
|  | b. | State and significance of LMTD. | CO1 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Describe in detail about principle and theory of molecular diffusion with neat sketch. | CO3 | 10 |
|  | b. | Differentiate between molecular diffusion and Eddy diffusion. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive the application of dimensional analysis for a forced convection equation h = f (ρ,u,l,μ,Cp,k). | CO3 | 10 |
|  | b. | Derive the application of dimensional analysis for a forced convection equation h = f (l,ρ,μ,k,Cp,βg,ΔT). | CO3 | 10 |