Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # UNIT OPERATIONS IN FOOD PROCESS ENGINEERING-I | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Describe the working of flat bed and deep bed dryers with diagrams. | CO1 | 10 |
|  | b. | Determine the total time required for drying period for the data given below: The food material is dried from 28 to 6% moisture content (wb) Weight of dry solids (Wd) = 200 kg Weight of dry solids per unit drying area = 20 kg / m2 Moisture content at the end of constant rate period(XC) = 0.23 kg moisture/kg of dry solids Rate of drying at the end of constant rate period(RC)=1.41 kg/m2 hr Rate of drying at the end of falling rate period(R2) = 0.082 kg/m2 hr | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the working of cabinet dryer with a neat figure. | CO2 | 10 |
|  | b. | Determine the values of “c” and “n” from Henderson’s equation for the following data. RH (%); t (℃ ); Me (%). 30; 50; 8.5. 70; 50; 14.5. | CO3 | 10 |
| 3. |  | Illustrate the principle and working of continuous rotary vacuum filter with a neat sketch. Enlist the applications of filtration in food processing. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Derive the basic equation for finding the terminal velocity of a particle in a settling tank. | CO1 | 10 |
|  | b. | Draw the figure of tubular centrifuge and explain its working. Write the applications of centrifugation in food processing. | CO2 | 10 |
| 5. |  | Draw neat figure of i) long tube rising film evaporator and ii) falling film evaporator indicating the various components and demonstrate the working process. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain the different types of feeding methods in triple effect evaporators with suitable figures. | CO2 | 20 |
| 7. | a. | Discuss the various types of size reduction methods. | CO1 | 10 |
|  | b. | Sugar crystals were ground from an average diameter of 650 µm to powder with an average diameter of 100 µm. The net energy consumption was 0.5 kWh per ton. Calculate the net energy consumption for grinding the crystals to 50 µm powder. Apply Rittinger’s and Kick’s laws. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the working of different types of disc attrition mills with suitable diagrams. | CO2 | 10 |
|  | b. | A food sample of 50 g was ground in a domestic grinder having a capacity of 555 watts. The time taken for grinding is 25 seconds. After grinding the final particle size was evaluated by sieve analysis and was found to be 0.52 mm. The initial particle size was 5 mm. Determine Bond’s work index (Wi). | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe the working of kneader with a neat sketch and enumerate the applications of mixing in food processing. | CO2 | 10 |
|  | b. | A vitamin premix of 1 kg is added to 999 kg of a food supplement. After mixing for 10 minutes in a blender, three samples each of 100 grams were collected and analyzed for the vitamin premix. The results of each sample in grams are : 0.12, 0.1, 0.13. Find the standard deviation, mixing index and time required for proper mixing for a standard deviation of 0.01. | CO3 | 10 |