Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # UNIT OPERATIONS IN FOOD PROCESS ENGINEERING - I | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss in detail the fluidization process in a fluidized bed dryer with suitable diagrams. | CO2 | 10 |
|  | b. | Determine the equilibrium moisture content of bajra at RH = 10% and t = 60 ℃ using Henderson’s equation where c = 6.15 x 10-6 and n = 2.25. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the principle and working of spray dryer with a neat sketch. | CO1 | 10 |
|  | b. | Calculate the amount of moisture evaporated from 500 kg of grain for drying it from an initial moisture content of 27% (wet basis) to a final moisture content of 12% (wet basis). Apply both wet basis and dry basis methods. | CO2 | 10 |
| 3. |  | Draw the figure and explain the working of the following centrifuges. a. Tubular centrifuge. b. Basket type centrifuge. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Derive the formula for the following during the separation of liquids in a centrifuge: a. Pressure acting in the centrifuge due to the liquid(ΔP). b. Radius of neutral zone (rn). c. Residence time of the particle in the centrifuge (tr). | CO2 | 20 |
| 5. | a. | Derive equations for material and energy balances in single effect evaporator. | CO1 | 10 |
|  | b. | An aqueous food solution at a temperature of 18℃ contains 8% solids by mass is to be concentrated to 30% solids in a single effect evaporator. The evaporator has a total heat transfer surface area of 10m2, uses steam at 300 kPa and operates under a vacuum of 79.3 kPa. The overall heat transfer coefficient to be 2500 W/m2 K. The various specific enthalpies and temperatures obtained from the steam tables are given below: Enthalpy of steam (hS) = 2725 kJ/kg Enthalpy of condensate (hC) = 561 kJ/kg Enthalpy of feed (hF) = 75.5 kJ/kg Enthalpy of vapour (hV) = 2613 kJ/kg Enthalpy of concentrated liquor (hL) = 260 kJ/kg. The temperature of steam (TS) at 300 kPa is 135℃ and the temperature of the saturated liquid water (TE) at the evaporator at a pressure of 79.3 kPa is 60.5℃. Determine the mass flow rate of steam required and the evaporator economy. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. |  | Sketch neat diagrams of short tube vertical and short tube horizontal evaporators. Explain their working and also mention the merits of the evaporators. | CO2 | 20 |
| 7. | a. | Explain the theory of mixing. | CO1 | 10 |
|  | b. | Draw a neat diagram of planetary mixer and explain its working. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the principle and working of ribbon mixer with a neat diagram. | CO2 | 10 |
|  | b. | During the preparation of a dough, 2 kg of sugar was mixed with 98 kg of flour in a sigma mixer. After 10 minutes of mixing three samples were taken and analysed for the percentage of sugar in the mixture. The results are : 0.75, 0.78, 0.73. Calculate the mixing index and the time required for perfect mixing. Consider σα = 0.01. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the working of hammer mill with suitable diagram. | CO2 | 10 |
|  | b. | Food grains of average initial size 6 mm were reduced to 0.0012 mm size using a 10 hp motor. Check whether this motor would be adequate to reduce the size of the particles to 0.0006 mm. Assume Rittinger equation and 1 hp = 745.7 watts. | CO3 | 10 |