Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14FP2010 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | UNIT OPERATIONS IN FOOD PROCESS ENGINEERING-II | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw and explain the steam distillation process with a neat sketch. | CO1 | 10 |
|  | b. | Write the applications of extraction and leaching in food industries. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Draw and explain the fractionating column with a neat diagram. | CO2 | 10 |
|  | b. | Steam distillation process is used for separating out a spice oil from the mint leaves by passing steam into the chamber at 70 kPa at the rate of 300 kg/hour for 30 minutes. If the molecular weight of the volatile oil and steam are 160 and 20 respectively, find the quantity of spice oil distilled. Consider the total pressure as 101.3 kPa. | CO3 | 10 |
| 3. | a. | Draw and describe the multi stage extraction process with diagram. | CO2 | 10 |
|  | b. | Give a detailed note on the applications of extraction in food processing. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Deduce the equation for equimolar diffusion of binary gases. | CO2 | 10 |
|  | b. | The natural food colour from cactus cladode is to be extracted with water. The saturated concentration of the colour in the water is found to be 5 kg/m3. In a laboratory scale extractor containing about one litre volume, it has taken 30 minutes to extract the colour from cladode to an extent of 0.975 kg/m3. Under similar conditions in a commercial plant of 15 m3 capacity, it is desired to extract 15 kg of the colour into the water. How much time does it take? | CO2 | 10 |
| 5. | a. | Describe the construction and the working of scraped surface and forced circulation evaporator with a sketch. | CO1 | 15 |
|  | b. | List out the various applications of crystallization in food processing. | CO3 | 5 |
| (OR) |  |  |  |  |
| 6. |  | Elaborate the adsorption equipments with a sketch. | CO2 | 20 |
| 7. |  | Dramatize the super critical fluid extraction process with a neat sketch and mention their application and limitation. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | List out the various applications of membrane separation process. | CO2 | 6 |
|  | b. | Enlist the various factors affecting rate of leaching. | CO2 | 7 |
|  | c. | Estimate the osmotic pressure of orange juice with 15% total solids at 20℃. The density of orange juice is 1048 kg/m3, Gas constant = 8.314 (m3 kPa/kg mol K) and molecular weight is 180 kg/kg mol. | CO2 | 7 |
|  |  | Compulsory: |  |  |
| 9. |  | Summarize the different types of membranes used in food processing and compare the characteristics of each membrane. | CO2 | 20 |