Reg.No. ____________

End Semester Examination – April/May– 2018

| # Code : | # 14FP2016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PHYSICAL PROPERTIES OF FOOD MATERIALS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Illustrate with a neat diagram the estimation of Radius of curvature. | CO1 | 8 |
|  | b. | Explain in detail about particle size distribution. | CO2 | 8 |
|  | c. | The major diameter (L) and the average of the minor and major diameters (H) of Wheat are measured as 8.76 mm and 2.83 mm, respectively. Calculate the minimum and maximum radii of curvature for the barley. | CO3 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Describe with neat sketches the method of determination of Volume and Density of : - Vegetables - Fruits | CO1 | 10 |
|  | b. | Rapeseeds are commonly used for volume measurement of baked goods. In a bakery, ﬁrst the bulk density of rapeseeds was determined by ﬁlling a container of 100 g weight and 1000 cm3 volume uniformly with rapeseeds through tapping and smoothing the surface with a ruler. The weight of the container ﬁlled with rapeseeds after constant weight was reached was 750 g. Then, the volumes of the mufﬁns were measured at different baking times using rapeseeds before and after compressing the pores using a 1000 N load cell and the experimental data are given in Table Calculate the porosity of mufﬁns during baking. How does it change during baking? Explain the reason. | CO2 | 10 |
| 3. | a. | Enumerate the applications of viscosity in food engineering. | CO2 | 5 |
|  | b. | Describe any one method for dough quality control. | CO2 | 8 |
|  | c. | Two parallel plates are 0.1 m apart. The bottom plate is stationary while the upper one is moving with a velocity V. The ﬂuid between the plates is water, which has a viscosity of 1 cp. - Calculate the momentum ﬂux necessary to maintain the top plate in motion at a velocity of 0.30 m/s. - If water is replaced with a ﬂuid of viscosity 100 cp, and momentum ﬂux remains constant, find the new velocity of the top plate. | CO3 | 7 |
| (OR) |  |  |  |  |
| 4. | a. | Dry commercial semolina ﬁbers with a diameter of 1.65 mm are used to examine the rheological properties of dry spaghetti. - A tensile test is applied on ﬁbers of 150 mm length and the result is given in Fig. - Determine the value of the modulus of elasticity. - What is the Poisson ratio if the ﬁbers exhibit a diameter change of 2.43 × 10−3 mm under the stress of 15 MPa? . | CO3 | 10 |
|  | b. | Give a brief note on classification of Rheology and explain about various types of fluids based on viscosity. | CO2 | 10 |
| 5. | a. | Derive an expression for thermal conductivity of gases from the principles of Kinetic Theory of gases. | CO2 | 10 |
|  | b. | The composition of a tropical fruit and the densities of food components are given below. Determine the thermal conductivity of the fruit at 30◦C, using parallel, series models. Component; Weight(%); Density (kg/m3). Water; 22.5; 995.7. Carbohydrate; 72.9; 1592.9. Protein; 2.2; 1319.6. Fat; 0.5; 917.15. Ash; 1.9; 2418.2. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the Transient hot wire methods available for determination of thermal conductivity of foods. | CO2 | 10 |
|  | b. | Describe the modified Fitch protocol. | CO1 | 10 |
| 7. | a. | Describe the role of Water activity in shelf life of foods. | CO2 | 10 |
|  | b. | Explain any one method of estimation of water activity. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Explain in detail about the following: - Osmotic pressure. - Freezing point depression. - Boiling point elevation. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Write a note on dipolar rotation. | CO2 | 5 |
|  | b. | Write a detail note on microwave heating. | CO3 | 5 |
|  | c. | Describe the effects of Temperature on dielectric constant and loss factor. | CO1 | 10 |