Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2021 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FOOD PROCESS EQUIPMENT DESIGN | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Illustrate with a neat diagram the construction and working of the following: - Cabinet dryer. - Puff drying. | CO3 | 10 |
|  | b. | Write a note applications of dryers in food industries. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Carrots with 85 per cent moisture content on wet basis is dried to a final moisture content of 10 percent. If we dry 200 kg of Carrots, what is the final weight of the product, and how much water is removed? | CO1 | 10 |
|  | b. | Derive an expression for mass and energy balance of a dehydration system. | CO2 | 10 |
| 3. | a. | Explain in detail about design of single effect evaporator. | CO3 | 15 |
|  | b. | Compare and contrast natural circulation and forced circulation evaporators. | CO3 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | A manufacturer is to design a heat exchanger in which the specification takes some steam at 4 bar g to heat secondary water from 10°C to 60°C . It is assumed that 1 litre of water has a mass of 1 kg. The manufacturer uses a Heat Transfer Coefficient 'U' for the heat exchanger of 2500 W/m2°C. Take the Specific Heat of water (cp) as 4.19 kJ/kg°C = 4190 J/ kg°C. Determine: - The design heat load. - The corresponding steam flow rate. - The minimum heating area required. | CO3 | 10 |
|  | b. | Calculate a preliminary estimate of the heat exchanger area needed to cool 55,000 lb/hr of a light oil (specific heat = 0.74 Btu/lb.°F) from 190°F to 140°F using cooling water that is available at 50°F. The cooling water can be allowed to heat to 90°F. An initial estimate of the Overall Heat Transfer Coefficient is 120 Btu/hr ft² °F. Also estimate the required mass flow rate of cooling water | CO1 | 10 |
| 5. | a. | The reaction A→B is to be carried out isothermally in a continuous-flow reactor. Calculate the CSTR,PFR volume to consume 79% of A, when the entering molar flow rate is 5 mol A/h, the volumetric flow rate is constant at 10 lit/h and the rate is –rA=(3 lit/mol•h)CA2. | CO3 | 10 |
|  | b. | Give the classification of pressure vessels. | CO2 | 5 |
|  | c. | Write a mass balance over batch reactor. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Explain in detail the Stresses in a Thin Cylindrical Shell due to an Internal Pressure. | CO1 | 10 |
|  | b. | A hydraulic press has a maximum capacity of 1000 kN. The piston diameter is 250 mm. Calculate the wall thickness if the cylinder is made of material for which the permissible strength may be taken as 80 MPa. This material may be assumed as a brittle material. | CO3 | 10 |
| 7. | a. | Describe in detail about the stainless steel used for construction of food processing equipments. | CO2 | 15 |
|  | b. | Explain the importance polymer coating in food equipments. | CO1 | 5 |
| (OR) |  |  |  |  |
| 8. |  | Explain the steps involved in design of food engineering equipments. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the importance and role of roofing types available for tanks. | CO2 | 10 |
|  | b. | Describe with a neat sketch the parts of any one storage tank and its features. | CO2 | 10 |