Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2024 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MECHANICAL SYSTEMS FOR FOOD PROCESSING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | The internal and external diameters of the impeller of a centrifugal pump are 200 mm and 400 mm respectively. The pump is running at 1200 r.p.m. The vane angles of the impeller at inlet and outlet are 20° and 30° respectively. The water enters the impeller radially and velocity of flow is constant. Determine the work done by the impeller per unit weight of water. Draw the velocity triangle for the given data. | CO, CO3 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Describe the construction, working, advantages and applications if Persitaltic pump. | CO1 | 15 |
|  | b. | Compare positive displacement pumps and dynamic pumps. | CO1 | 5 |
| 3. | a. | Determine the diameter of a solid shaft which will transmit 90 kW at 160 rpm if the shear stress in the shaft is limited to 60 N/mm2. Find also the length of the shaft, if the twist must not exceed 1 degree over the entire length. Take C = 8 x 104 N/mm2. | CO2, CO3 | 15 |
|  | b. | Enlist the types of chains used in power transmission and describe their design. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Give the design and working of Fast and loose pulley drive with a neat diagram. | CO2 | 10 |
|  | b. | Draw a neat sketch of universal coupling showing the various pins and joints. Also write short notes about the coupling. |  | 10 |
| 5. |  | Two pulleys one 450mm diameter and the other 200mm diameter are on parallel shafts l .95m apart and are connected by cross belt. Find the length of belt and angle of contact between the belt and each pulley. What power can be transmitted by the belt when the larger pulley rotates at 200 rev/min, if the maximum permissible tension in the belt is 1 kN, and the coefficient of friction between the belt and pulley is 0.25? | CO2, CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Define and describe the working of the following boiler elements with a neat diagram i) Dead Weight Safety Valve. ii) Fusible Plug. | CO2 | 10 |
| 7. |  | Write in detail about types of equipment used, operation and application in food industries of the following: i) Liquid Freezing. ii) Air Freezing Equipment. | CO2 | 10+10 |
| (OR) |  |  |  |  |
| 8. |  | A refrigerator using NH3 works between the temperatures -100°C and 250°C.The gas is dry at the end of compression and there is no undercooling of liquid. Calculate the theoretical C.O.P. of the cycle. The properties of NH3 are given below: Temperature (0C); Liquid heat (hf) (kJ/kg); Liquid heat (hfg) (kJ/kg); Liquid entropy(sf) (kJ/kg). 25 -10; 298.9 135.37; 1166.94 1297.8; 1.1242 0.5443. | CO2,CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the construction, operational mechanics and demonstrate the applications the following with a neat sketch: i) Belt Conveyors. ii) Bucket Conveyors. | CO2 | 10+10 |