Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14FP2025 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CEREALS AND PULSES TECHNOLOGY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Discuss the various parboiling methods and indicate the effect of parboiling. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Dramatize the whitening of brown rice with a neat sketch. | CO1 | 20 |
| 3. | a. | Draw and describe the construction and working of rotary dryer. | CO1 | 10 |
|  | b. | Point out the various unit operations carried out in pulse milling. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | List out the various factors affecting pulse milling out turn and state the formula used for finding pulse milling efficiency. | CO2 | 12 |
|  | b. | In pigeon pea milling experiment with concentric cylinder abrasive the following observations were made - Amount of unhulled grains – 3% - Recovery whole split kernels after milling – 72% - Amount of crushed kernels – 3% - Amount of powder generated – 11% - Amount of husk removed – 11% The cotyledon to grain ratio of the grain was 86%. Calculate the milling efficiency of the system. (Theoretical husk content – 14%) | CO2 | 8 |
| 5. | a. | Summarize the wet milling methods of corn processing using a neat flow chart. | CO2 | 14 |
|  | b. | Paddy weighing 860 kg/m3 is loaded in a circular concrete silo of 3.5 m internal diameter and a clear height of 6m. The angle of internal friction for paddy is 35° and that for paddy and concrete is 25°. Apply Airy’s theory and find out the maximum lateral pressure at the bottom of bin section. | CO3 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Express your idea on various improved storage structure with a diagram. | CO3 | 12 |
|  | b. | Compile your idea on moisture migration in storage structure with a neat sketch. | CO3 | 8 |
| 7. |  | A RCC cylindrical grain storage bin has an internal diameter of 5m and is 20m deep. It is completely filled with wheat weighing 740kg/m3. The angle of internal friction for wheat is 30° while the angle of friction between wheat and bin wall is 25°. The ratio of horizontal and vertical pressure intensity ‘k’ is 0.4. Calculate the lateral pressure intensity at 2m intervals. Tabulate the results at various depths. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Design a bag storage structure for storing 450 tonnes of paddy. | CO3 | 15 |
|  | b. | Write short notes on fumigation. | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | Generalize the concept of bucket elevator and discuss the design of various parts of the elevator with a neat diagram. | CO3 | 20 |