Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENGINEERING MECHANICS | # Max. marks : | # 100 |

# (for civil only)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Five forces are acting on particles. The magnitudes of the forces are 300N, 600N, 700N, 900N and P and their respective angles with the horizontal are 0o, 60o, 135o, 210o and 270o. If the vertical component of all the forces is -1,000m, find the value of P. Also calculate the magnitude and the direction of the resultant, assuming that the first force acts towards the point, while all the remaining forces act away from the point. X Y 60O 600N 900N 30O 700N 45O P 300N Fig. 1 | CO1 | 8+12 |
| (OR) |  |  |  |  |
| 2. |  | Two identical rollers, each of weight 50N, are supported by an inclined plane and a vertical wall as shown in figure 1 below. Find the reactions at the points of supports A, B and C. Assume all the surfaces to be smooth. A B 30O C Fig. 1 | CO1 | 20 |
| 3. |  | Compute the simplest resultant of the parallel force system shown in figure 3 below: 25N 10N 40N 15N 1m 2m 1m A C D B Fig. 3 20N 1m E | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | The 3 cables are secured to a ring at B and the turn buckle at C as shown in fig.4 is tightened until it supports at tension of 1.6kN. Calculate the moment M, produced by the tension in the cable AB about the base of mast at D. 30o 45o 30o B A C C 10m Fig. 4 | CO2 | 20 |
| 5. |  | A simply supported beam is loaded as given in figure 3. Determine the reactions at the supports. Fig. 5 | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Find the moment of inertia of plane area shown in fig. 6 about its centroidal axis. 0.9cm Fig. 6 2cm 2.1cm 1.2cm 5.0cm 1.8cm B A | CO2 | 20 |
| 7. |  | A bowling ball is dropped from a boat so that it strikes the surface of the lake with a speed of 8m/s. Assuming the ball experiences a downward acceleration of a = 3 – 0.1 v2, (where a and v are expressed in m/s2 and m/s respectively) when the water, determine the velocity of the ball when it strikes the bottom of the lake. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | What should be the value of ɵ in figure below, so that the motion of the 90N block impends down the plane? Assume the coefficient of friction μ for all the surfaces is 1/3. 90 N 30 N | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | A car of mass 300kgs is travelling at a speed of 36 km/hr on a level road. It is brought to rest, after travelling a distance of 5m.What is the average force of resistance acting on the car. | CO3 | 14 |
|  | b. | A bullet of mass 25 gms is moving with a velocity of 500m/s and fired into a body of 12kg, which is suspended by a string, fixed at top, 1m long. The bullet gets embedded into the body and the unit (bullet + body) swings through some angle. Find the angle through which the unit swings. | CO3 | 6 |