Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code: | # 14ME2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # ENGINEERING MECHANICS | # Max. Marks: | # 100 |

(for Mech Only)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | State parallelogram law of forces. | CO1 | 5 |
|  | b. | Two cables are tied together at C and are loaded as shown. Determine the tension i) in cable AC, ii) in cable BC. Where α is 20°. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2 | a. | Two rollers, each of weight 50N and of radius 10cm rest in a horizontal channel of width 36cm as shown below. Find the reaction on the point of contacts A, B and C. | CO1 | 15 |
|  | b. | List the different types of system of forces. | CO1 | 5 |
| 3. | a. | The tension in cables AB and AC are 100N and 120N respectively as shown in figure below.Determine the magnitude of the resultant force acting at A. | CO1 | 15 |
|  | b. | Explain the conditions of equilibrium? | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Determine the supports and reactions of the given figure. | CO2 | 15 |
|  | b. | State Varignon’s theorem. | CO1 | 5 |
| 5. |  | Find the moment of inertia of a unsymmetrical I section about its centroid axes having top flange as 60mm *20mm, bottom flange as 100mm*20mm and web is 60mm*20mm. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Locate centroid of the sectioned area shown in figure. | CO2 | 20 |
| 7. | a. | A block of weight 150N is resting on a rough inclined plane as shown in the fig. The block is tied up by a horizontal string, which has a tension of 50N.Find (i) the frictional force on the block (ii) the normal reaction of the inclined plane (iii) the co-efficient of friction between the surfaces of contact. | CO3 | 15 |
|  | b. | Write short notes on cone of friction. | CO4 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Two blocks A (mass 28kg), B (mass 28 kg) are separated by 12 m as shown in the figure. If the blocks start moving, find the time t when the blocks collide. Assume µ = 0.25 for block A and plane and µ = 0.1 for block B and plane. | CO3 | 15 |
|  | b. | State parallel axis theorem. | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | A uniform ladder weighing 100N and 5 meters long has lower end B resting on the ground and upper end A resting against a vertical wall as shown in fig. The inclination of the ladder with horizontal is 60°.if the coefficient of the friction at all surfaces of contact is 0.25; determine how much distance up long the ladder a man weighing 600N can ascent without causing it to slip. | CO4 | 20 |