Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENGINEERING MECHANICS | # Max. marks : | # 100 |

(for civil only)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Five forces are acting on a particles. The magnitudes of the forces are 300N, 600N, 700N, 900N and P and their respective angles with the horizontal are 0o, 60o, 135o, 210o and 270o. If the vertical component of all the forces is -1,000m, find the value of P. Also calculate the magnitude and the direction of the resultant, assuming that the first force acts towards the point, while all the remaining forces act away from the point. X Y 60O 600N 900N 30O 700N 45O P 300N Fig. 1 | CO1 | 8 12 |
| (OR) |  |  |  |  |
| 2. |  | Two rollers, each of weight 50N and of radius 100m rest in a horizontal channel of width 360mm as shown in fig.2 below. Find the reaction on the points of contacts A, B and C. 50N 50N 1 2 C A B 360mm Fig. 2 | CO1 | 20 |
| 3. |  | Compute the simplest resultant of the parallel force system shown in figure 3 below: 15N 40N 30N 20N 0.5m 1m 1m A C D B Fig. 3 | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Determine the magnitude and line of action of the resultant of forces as shown in fig. 4 below. 45o 30o 40kN 15kN 50kN 3m 3m A B C D Fig. 4 Fig.4 | CO2 | 20 |
| 5. |  | A simply supported beam is loaded as given in figure 5. Determine the reactions at the supports. 30kN 20kN 20kN 10kN/m B A 3m 2m 3m 2m Fig. 5 | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Find the moment of inertia of plane area shown in fig. 6 about its centroidal axis. 40mm 20mm 40mm 20mm 120mm X Y O Fig. 6 | CO2 | 20 |
| 7. |  | A bowling ball is dropped from a boat so that it strikes the surface of the lake with a speed of 8m/s.Assuming the ball experiences a downward acceleration of a = 3 – 0.1 v2, (where a and v are expressed in m/s2 and m/s respectively) when the water, determine the velocity of the ball when it strikes the bottom of the lake. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A boy throws two stones at an angle of 30o from point A as shown in fig 8 below. Determine the time between throws so that both stones strike the edges of the tank B and C at the same instant. With what speed must he throw stone? TANK 1m 2.5m 4m B C A u2 u1 0.25m 1 2 x y Fig. 8 | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Two blocks A and B of weight 120N and 100N are hung to the ends of a rope, which is passing over an ideal pulley as shown in fig.9a. The velocity of the system is increased from 1m/s to 2m/s. How much the distance, these blocks will move? Also calculate the tension in the string? Use work – energy method. Fig. 9a A B 120N 100N | CO4 | 14 |
|  | b. | A bullet of mass 25 gms is moving with a velocity of 500m/s and fired into a body of 12kg, which is suspended by a string, fixed at top, 1m long. The bullet gets embedded into the body and the unit (bullet + body) swings through some angle. Find the angle through which the unit swings. | CO4 | 6 |