Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 14ME2005 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : MACHINING PROCESSES |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | What do you mean by tool life: How it is specified? With appropriate sketches and graphs, discuss the various factors affecting tool life: | CO1 | 14 |
|  | b. | In an orthogonal cutting if the depth of cut is 1.50 mm and chip thickness after cutting is 2 mm; find the following i) chip thickness ratio ii) shear angle and iii) friction angle. The tool bit has a rake angle of 10°. Assume the coefficient of friction as 0.9. | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. |  | Explain the functions, desirable properties and types of cutting fluids used in metal cutting. | CO1 | 20 |
| 3. |  | Explain the constructional features and specifications of centre lathe with neat sketch. Also explain the different lathe accessories. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | What is the importance of the quick return mechanism in a shaper machine? Explain any two types of quick return mechanisms with neat sketches. | CO1 | 20 |
| 5. |  | Draw a neat sketch of drill bit and explain all the terminology used for nomenclature. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | With a neat sketch, explain the constructional features and specifications of column and knee type milling machine. | CO1 | 20 |
| 7. |  | Write short notes on (a) Honing (b) lapping (c) super finishing (d) polishing and buffing. | CO1 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Describe any two types of gear generation methods. | CO1 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the designation of grinding wheels. Also explain the factors influencing the selection of grinding wheels for various applications. | CO1 | 20 |