Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | 14ME2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FLUID POWER CONTROL ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | List the applications of fluid power. | CO1 | 4 |
|  | b. | Identify the essential components of pneumatics system with neat diagram. | CO1 | 12 |
|  | c. | Outline the advantages of pneumatic power systems and hydraulic systems over electrical systems. | CO1 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Sketch and discuss the working principle for the following hydraulic motors : i) External gear motor ii) Vane motor. | CO2 | 14 |
|  | b. | A pump has a displacement of 81.9cm3. The pump delivers 75.8x10-3 m3/min. at 1000 rpm at 67 bar. If the prime mover input torque is 100 Nm. Determine overall, volumetric and mechanical efficiencies. | CO2 | 6 |
| 3 | a. | Draw various directional control valve spool switching mechanisms | CO3 | 10 |
|  | b | Appraise the function of intensifier and discuss with a hydraulic circuit. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Describe various types of pressure accumulators with neat sketches. | CO3 | 10 |
|  | b. | Draw two types of accumulator circuits and discuss the working. | CO3 | 10 |
| 5. |  | Build the Synchronizing hydraulic circuits with - Series piping - Tie rod cylinders - Matching Pumps - Flow Control Valves - Flow dividing Motors. | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | How solenoid will work, explain with neat diagram. | CO2 | 10 |
|  | b. | Analyse the working of relay with a practical example. | CO2 | 10 |
| 7. | a. | Design and develop a pneumatic circuit for the sequence of A+B+A-B-. | CO4 | 12 |
|  | b. | Design and develop a electro pneumatic circuit using limit switch for double acting cylinder | CO3 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | With a neat sketch, explain the hydraulic circuit for quick return motion of surface grinding machine table movement. | CO4 | 10 |
|  | b. | Draw the pneumatic circuit for the use of time delay. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Summerise the working of following fluidic sensor with neat sketch - Coanda Effect - OR/NOR and AND/NAND monostable devices - Cone-Jet Sensor - Interruptible Jet Sensor - Reflex Proximity Switch | CO4 | 20 |