Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENGINEERING THERMODYNAMICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | A piston cylinder machine contains a fluid system which passes through a complete cycle of four processes. During a cycle, the sum of all heat transfer is -170 kJ. The system completes 100 cycles per minute. Complete the following table showing the method for each item, and compute the net rate of work output in kW. Process; Q (kJ/min); W (kJ/min); ΔE (kJ/min). a - b; 0; 2170; ---. b - c; 21000; 0; ---. c – d; -2100; ---; -36600. d - a; ---; ---; ---. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | In a gas turbine, the gas enters at the rate of 5 kg/s with a velocity of 50 m/s and enthalpy of 900 kJ/kg and leaves the turbine with a velocity of 150 m/s and enthalpy of 400 kJ/kg. The loss of heat from the gases to the surroundings is 25 kJ/kg. Assume for gas R = 0.285 kJ/kgK and Cp = 1.004 kJ/kgK and the inlet conditions to be at 100 kPa and 27°C. Determine the power output of the turbine and diameter of the inlet pipe. | CO1 | 20 |
| 3. |  | A reversible heat engine operates between two reservoirs at temperatures of 600˚ C and 40˚C. The engine drives a reversible refrigerator which operates between reservoirs at temperatures of 40˚C and 20˚C. The heat transfer to the heat engine is 2000kJ and the net work output of the combined engine refrigerator plant is 360kJ. Evaluate the heat transfer to the refrigerant and the heat transfer to the reservoir at 40˚ C. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | State and explain the Carnot’s theorem with suitable block diagram. | CO1 | 10 |
|  | b. | A refrigeration plant for a food store maintains a temperature of – 10C. The ambient air temperature is 32C. If heat leaks into the plant at the continuous rate of 2 kJ/s, what is the least power necessary to pump this heat out continuously? | CO1 | 10 |
| 5. | a. | Draw and explain the phase equilibrium diagram for a pure substance on p-v coordinates. | CO2 | 10 |
|  | b. | Draw and explain the phase equilibrium diagram for a pure substance on p-T coordinates. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | One kg of water at 273 K is brought into contact with a heat reservoir at 373 K. When the water has reached 373 K, find the entropy change of the water, the reservoir and the universe. | CO2 | 10 |
|  | b. | If water is heated from 273 K to 373 K by first bringing it in contact with a reservoir at 323K and then with a reservoir at 373 K , what will be the entropy change of the universe? | CO2 | 10 |
| 7. |  | A vessel having a capacity of 0.04 m3 contains a mixture of saturated water and saturated steam at a temperature of 250°C. The mass of the liquid present is 9 kg. Find the following: - The pressure. - The mass. - The specific volume. - The specific enthalpy. - The specific entropy. - The specific internal energy. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A mixture of ideal gases consists of 3 kg of nitrogen and 5 kg of carbondioxide at a pressure of 300 kPa and a temperature of 200C. find - The mole fraction of each constituent - The equivalent molecular weight of the mixture - The equivalent gas constant of the mixture - The partial pressure and partial volumes - The volume density of the mixture - The cp and cv of the mixture If the mixture is heated at constant volume to 400C, find the changes in internal energy, enthalpy and entropy of the mixture. Take γ for CO2 and N2 to be 1.286 and 1.4 respectively. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Atmospheric air at 1.0132 bar has a DBT of 320C and WBT of 260C. Compute - The partial pressure of water vapour - The specific humidity - The dew point temperature - The relative humidity - The degree of saturation - The density of the air in the mixture - The density of vapour in the mixture - The enthalpy of the mixture. | CO4 | 20 |