Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ENGINEERING THERMODYNAMICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

#### (Use of Steam Tables, Refrigeration Table and Compressibility chart are permitted)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A system is taken through a series of processes as a result of which it is restored to the initial state. The work and heat interaction for some of the processes are measured and given below. Complete the table and determine the net work done and net heat interaction. Process; W(kJ); Q(kJ); ΔU(kJ). 1-2; 100; 200; ?. 2-3; ?; 150; 200. 3-4; 250; ?; ?. 4-1; 300; ?; 50. | CO1 | 10 |
|  | b. | Air at 1.02 bar, 22˚C initially occupying a cylinder volume of 0.015 m3, is compressed isentropically by a piston to a pressure of 6.8 bar. Calculate the final temperature, final volume and work done. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Derive the steady flow energy equation applied to compressor and nozzle. | CO1 | 5 |
|  | b. | In a gas turbine the gas enters at the rate of 5 kg/s with a velocity of 50 m/s and enthalpy of 900 kJ/kg and leaves the turbine with a velocity of 150 m/s and enthalpy of 400 kJ/kg. The loss of heat from the gases to the surroundings is 25 kJ/kg. Assume for gas R = 0.285 kJ/kg K and Cp = 1.004 kJ/kg K and the inlet conditions to be at 100 kPa and 270C. Determine the power output of the turbine and the diameter of the inlet pipe. | CO1 | 15 |
| 3. | . | Two reversible heat engines A and B are arranged in series, A rejecting heat directly to B. Engine A receives 200kJ at a temperature of 421°C from a hot source, while the engine B is in communication with a cold sink at a temperature of 4.4 °C. If the work output of A is twice that of B. Find i. The intermediate temperature between A and B ii. The efficiency of each engine iii. The heat rejected to the cold sink | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | A fluid undergoes a reversible adiabatic compression from 0.5 MPa, 0.2 m3 to 0.05 m3 according to the law, pv1.3= constant. Determine the change in enthalpy, internal energy and entropy, and the heat transfer and work transfer during the process. | CO2 | 15 |
|  | b. | Explain Kelvin-Planck statement and Clausius statement of the second law. | CO1 | 5 |
| 5. | a. | Draw and explain the phase equilibrium diagram on P- coordinates for a substance which shrinks in volume on melting and indicates the relevant constant property lines. | CO3 | 10 |
|  | b. | A vessel of volume 0.04 m3 contains a mixture of saturated water and saturated steam at a temperature of 2500C. The mass of the liquid present is 9 kg. Find the pressure, the mass, the specific volume, the enthalpy, the entropy and the internal energy. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Draw and explain the phase equilibrium diagram for a pure substance on T-s plot with relevant constant property lines. | CO3 | 10 |
|  | b. | Steam initially at 1.5 MPa, 300C expands reversibly and adiabatically in a steam turbine to 40C. Determine the ideal work output of the turbine per kg of steam. | CO3 | 10 |
| 7. | a. | State Dalton’s law of partial pressure. | CO3 | 2 |
|  | b. | A gas mixture consists of 12kg of methane, 5 kg of nitrogen and 3 kg of oxygen. Determine the molecular mass and gas constant of the mixture. If the total pressure is 100 kPa. Calculate their partial pressures. | CO3 | 8 |
|  | c. | Air contains 21% of oxygen and 79% of nitrogen by volume. Determine the molecular weight and gas constant of air. | CO3 | 5 |
|  | d. | State the assumptions made in kinetic theory of gases. | CO3 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | State the equation of state for Vander Wall’s and explain the importance of each term. Also bring out the limitations of the equation. | CO3 | 8 |
|  | b. | Draw and explain the generalized compressibility chart. | CO3 | 6 |
|  | c. | A specific volume of CO2 at 1000C is 1 m3/kg. Determine the pressure exerted by CO2 i) using vanderwaal’s equation. Ii) using ideal gas . Take a=362850 Nm4/kg-mol2, b=0.0422 m3/kg-mol. | CO3 | 6 |
|  |  | Compulsory: |  |  |
| 9. | a. | Prove that specific humidity of air is | CO4 | 5 |
|  | b. | Air at 200C, 40%RH is mixed adiabatically with air at 400C, 40% RH in the ratio of 1kg of former with 2kg of the latter (on dry basis) find the final condition of air. | CO4 | 15 |