Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2015 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THERMAL ENGINEERING 1 | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | The following observation were recorded during a boiler trial: Pressure of steam = 10 bar, Steam condenser = 540 kg/hr Fuel used = 65 kg/hr, Moisture in fuel = 2% by mass, Mass of dry flue gases = 9 kg/kg of fuel, Lower calorific value of fuel = 32,000 kJ/kg Temperature of the flue gases = 325° C Temperature of boiler house = 28° C, Feed water temperature = 50° C , Mean specific heat of flue gases = 1 kJ/kg K, Dryness fraction of steam = 0.95, Specific heat of superheated steam = 2.1kJ/kgK. Draw up a heat balance sheet for the boiler. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the construction and working of a Lancashire boiler with the help of suitable sketches. | CO1 | 12 |
|  | b. | A coal fired boiler plant consumes 400 kg of coal per hour. The boiler evaporates 3200 kg of water at 44.5oC into superheated steam at a pressure of 12 bar and 274.5oC. If the calorific value of fuel is 32760 kJ/kg of coal, calculate equivalent evaporation from and at 100oC and thermal efficiency of the boiler. | CO1 | 8 |
| 3. |  | Derive an equation for the mass of steam discharged through nozzle and also the equation for the critical pressure ratio. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Dry saturated steam at a pressure of 8 bar enters a convergent-divergent nozzle and leaves it at a pressure of 1.5 bar, If the flow is isentropic and the corresponding expansion index is 1.135, find the ratio of cross-sectional area at exit and throat for maximum discharge. | CO2 | 12 |
|  | b. | Explain metastable flow of steam through a nozzle. | CO2 | 8 |
| 5. | a. | Draw the combined velocity triangle for an impulse turbine and derive the power produced by an impulse turbine. | CO3 | 15 |
|  | b. | Write the difference between impulse and reaction turbine? | CO3 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the term ‘compounding of steam turbine’ and discuss the pressure-velocity compounding of turbine with neat sketch. | CO3 | 12 |
|  | b. | Write the classifications of steam turbines. | CO3 | 8 |
| 7. |  | With neat sketches of T-s and p-v diagram for a single stage reciprocating air compressor without clearance, derive the expression for the workdone when compression is isothermal and isentropic. | CO4 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Determine the size of the cylinder for a double acting air compressor of 40kW indicated power, in which air in at 1 bar and 15°C and compressed according to the law pv1.2= constant, to 6 bar. The compressor runs at 100rpm with average piston speed of 152.5m/min. Neglect clearance. | CO4 | 10 |
|  | b. | Define i) single acting compressor ii) Single stage compressor iii) Volumetric efficiency iv) Displacement of the compressor v) Free air delivery. | CO4 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | With the neat sketch explain the working principle of vapour compression refrigeration system and show the thermodynamic processes on T-s and p-h diagram. | CO5 | 20 |