Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2015 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THERMAL ENGINEERING - 1 | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define boiler efficiency and factors on which it depends. | CO1 | 5 |
|  | b. | Th following data refer to a boiler plant consisting of an economizer,a boiler and a superheater. Mass of water evaporated per hour = 5940 kg,mass of coal burnt per hour = 675kg,L.C.V of coal = 31600 kJ/kg,pressure of steam at boiler stop valve = 14 bar,temperature of feed water entering the economizer = 32°C,temperature of feed water leaving the economizer = 115°C,dryness fraction of steam leaving the boiler and entering superheater =0.96,temperature of steam leaving the superheater = 260°C,specific heat of superheated steam = 2.33. 1) Determine percentage of heat in coal utilized in economizer,boiler and superheater: 2) Overall efficiency of boiler plant. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the heat losses in a boiler power plant. | CO1 | 10 |
|  | b. | The steam used by the turbine is 5.4kg/kWh at a pressure of 50bar and a temperature of 350°C.The efficiency of boiler is 82% with feed water at 150°C. - How many kg of 28100 kJ coal are required /kWh? - If the cost of coal/tonne is Rs.500,what is fuel cost/kWh? | CO1 | 10 |
| 3. | a. | Define critical pressure ratio for the nozzle of the steam turbine. Obtain analytically its value in terms of the index of expansion. | CO1 | 10 |
|  | b. | In a steam nozzle,the steam expands from 4 bar to 1 bar.The initial velocity is 60 m/s and the initial temperature is 200°C. Determine the exit velocity if the nozzle efficiency is 92%. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | What is the effect of friction on the flow through a steam nozzle? Explain with the help of h-s diagram. | CO1 | 10 |
|  | b. | Dry saturated steam enters a steam nozzle at a pressure of 15 bar and is discharged at a pressure of 2.0 bar. If the dryness fraction of discharge steam is 0.96, what will be the final velocity of steam? Neglect initial velocity of steam. If 10% of heat drop is lost in friction, find the percentage reduction in the final velocity. | CO1 | 10 |
| 5. | a. | Explain the pressure velocity compounding method to reduce the rotor speed with a neat sketch. | CO2 | 5 |
|  | b. | The velocity of steam exiting the nozzle of the impulse stage of a turbine is 400 m/s.Theblades operate close to the maximum blading efficiency. The nozzle angle is 20°. Considering equiangular blades and neglecting blade friction,calculate for a steam flow of 0.6kg/s,the diagram power and the diagram efficiency. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | Differentiate between impulse and reaction turbine. | CO2 | 5 |
|  | b. | A single stage steam turbine is supplied with steam at 5 bar, 200°C at the rate of 50kg/min.it expands into a condenser at a pressure of 0.2 bar. The blade speed is 400m/s. The nozzles are inclined at an angle of 20° to the plane of the wheel and the outlet blade angle is 30°.Neglecting friction losses, determine the power developed, blade efficiency, and stage efficiency. | CO2 | 15 |
| 7. | a. | Derive an expression for the shaft work of a reciprocating compressor assuming zero clearance volume. | CO3 | 5 |
|  | b. | A single stage,single acting reciprocating air compressor with 0.3m bore and 0.4m stroke runs at 400 rpm.The suction pressure is 1 bar at 300K and the delivery pressure is 5 bar.Find the power required to run it,if compression is isothermal,compression follows the law pv1.3= C and compression is reversible adiabatic.Also find the isothermal efficiency. | CO3 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Define volumetric efficiency of a reciprocating compressor. | CO3 | 5 |
|  | b. | Air is to be isentropically compressed at the rate 1 m3/s from 1 bar and 20°C to 10 bar.Find the work of compression and the volumetric efficiency if the clearance volume is 4% of stroke volume for all the cylinders for the following cases: - Single stage - Two stage and - Three stage compression.Assume density of air as 1.2 kg/m3. | CO3 | 15 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain vapour absorption refrigeration system with a neat sketch. | CO3 | 20 |