Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # THERMAL ENGINEERING II | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | # With a neat sketch explain the valve timing diagram of a four stroke petrol engine. | CO1 | 10 |
|  | b. | Explain pressure lubrication system of an I.C. Engine with a neat sketch. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | # The bore and stroke of a single cylinder, two stroke petrol engine are 12 cm and 17.31 cm, respectively. The clearance volume is 460 cc. Find the compression ratio and air standard efficiency of the cycle. If the relative efficiency is 55% and mechanical efficiency is 80%, find the specific fuel consumption on brake power basis. Calorific value of fuel used is 42,000 kJ/kg. | CO1 | 20 |
| 3. |  | Derive an expression for air standard efficiency of an Otto cycle. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | An ideal Diesel cycle has compression ratio of 16 with lowest temperature and pressure as 27C and 1 bar respectively. If heat added during the cycle is 500 kJ/kg of air, find pressure and temperature at each state point, work done in the cycle/kg, thermal efficiency of the cycle and mean effective pressure. | CO1 | 20 |
| 5. | a. | Explain year round air-conditioning system with a neat sketch. | CO4 | 10 |
|  | b. | Explain the equipment used in air-conditioning system. | CO4 | 10 |
| (OR) |  |  |  |  |
| 6. |  | A conference room for seating 50 persons is to be maintained at 22ºC dry bulb temperature and 60% relative humidity. The outdoor conditions are 40ºC dry bulb temperature and 27 º C wet bulb temperatures. The various loads in the auditorium are as follows: Sensible and latent heat loads per person 160 W and 100 W respectively; lights and fans 15000 W; sensible heat gain through glass, walls, ceilings etc. 15000 W. The air infiltration is 1200 m3/hr. Determine Room Sensible Heat Factor (RSHF). | CO4 | 20 |
| 7. |  | Air flowing in a duct has a velocity of 300 m/s, pressure 1.0 bar and Temperature 290 K. Taking  = 1.4 and R=287 J/kg K determine: - Stagnation pressure and temperature, - Velocity of sound in the dynamic and stagnation conditions, - Stagnation pressure assuming constant density. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Air at P0=10 bar and T0=400K is supplied to a 50 mm diameter pipe. The friction factor for the pipe surface is 0.002. If the Mach number changes from 3.0 at the entry to 1.0 at the exit determine i) the length of the pipe and ii) the mass flow rate. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the working principle of Turbojet Engine with a neat sketch. | CO3 | 10 |
|  | b | Explain open cycle constant pressure gas turbine with a neat sketch. | CO3 | 10 |