Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # POWER PLANT ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Differentiate a combined vapour power cycle from a binary one. Use T-s plots to illustrate your answer. | CO1 | 10 |
|  | b. | Explain how a Rankine cycle is modified in order to enhance the efficiency of thermal power plants. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Illustrate with a neat sketch the construction and operation of a high pressure boiler used in modern thermal power plants. | CO1 | 10 |
|  | b. | Steam at 90 bar and 480C is supplied to a steam turbine. Steam is reheated to its original temperature passing the steam through a reheater at 12 bar. The expansion after reheating takes place to a condenser pressure of 0.07 bar. Find the efficiency of reheat cycle and work output if the flow of steam is 1 kg/s. Neglect the pressure loss in the system and assume the expansion through the turbine is isentropic. Do not neglect the pump work. | CO2 | 10 |
| 3. | a. | Why are condensers necessary in thermal power plants? Illustrate the operation of any one type with a neat sketch. | CO1 | 10 |
|  | b. | Classify types of combustion equipment used in thermal power plants.With a neat sketch explain the construction and operation of a pulverized type combustion system. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Sketch and explain the operation of a gas turbine power plant incorporating a reheater, regenerator and an intercooler. | CO1 | 10 |
|  | b. | In a gas turbine the compressor takes in air at a temperature of 15oC and compresses it to four times the initial pressure with an isentropic efficiency of 82%. The air is then passed through a heat exchanger heated by the turbine exhaust before reaching the combustion chamber. In the heat exchanger 78% of the available heat is given to the air. The maximum temperature after constant pressure combustion is 600oC, and the efficiency of the turbine is 70%. Neglecting all losses except those mentioned, and assuming the working fluid throughout the cycle to have the characteristics of air and find the efficiency of the cycle. Assume R=0.287 kJ/kg K and =1.4 for air and constant specific heats throughout. | CO2 | 10 |
| 5. | a. | Differentiate a pressurized water nuclear reactor from a boiling water type. Illustrate with neat sketches. | CO1 | 10 |
|  | b. | Write detailed notes on the choice of materials used for the construction of nuclear power plants. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Sketch and label the components of a diesel power plant. What are its advantages? | CO1 | 10 |
|  | b. | With a neat sketch explain the working of hydroelectric power plants. | CO1 | 10 |
| 7. | a. | How can emissions from power plants pollute the atmosphere? Elaborate your answer with examples of pollutants. | CO4 | 10 |
|  | b. | Describe with sketches the processes of NOxand SOxremoval from flue gases. | CO4 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | How is a load curve drawn? | CO3 | 2 |
|  | b. | Define i) Diversity factor and ii) Capacity factor as applied to power plants. | CO3 | 4 |
|  | c. | The loads on a power plant with respect to time for 24 hours are tabled as follows: Time, Hrs; 0-6; 6 - 10; 10 - 12; 12 - 16; 16–20; 20 - 22; 22 - 24. Load, MW; 30; 70; 90; 60; 100; 80; 60. Construct the load curve and compute load factor of the power station. If the loads above 70 MW are taken by a stand-by unit of 30 MW capacity, find the load factor of the stand-by unit. | CO3 | 14 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain with a neat sketch the principle of electricity generation using a magneto hydro dynamic power plant. | CO1 | 14 |
|  | b. | How is electricity generation possible with i) wind ii) wave and iii) solar energy? Draw neat sketches. | CO1 | 6 |