Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2019 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name: | # HEAT AND MASS TRANSFER | # Max. marks: | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No | Sub Div | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive an expression for one dimensional heat conduction in a plane wall. | CO1 | 10 |
|  | b. | # A hollow sphere of pure iron contains a liquid chemical mixture that releases 30 kW of energy. The ID and OD of the sphere are 15cm and 30cm respectively. If the outer surface temperature of the sphere is 40ºC, determine the temperature at a location 2.5cm from the outer surface. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Derive the general three-dimensional heat conduction equation in Cartesian coordinates. | CO1 | 20 |
| 3. | a. | Derive an expression for internal heat generation in a cylinder. | CO1 | 15 |
|  | b. | A plane wall 10cm thick generates heat at the rate of 40000 W/m3 when an electric current is passed through it. The convective heat transfer coefficient between each face of the wall and the ambient air is 50 W/m2K. Determine the surface temperature and maximum temperature in the wall. Assume the ambient temperature to be 20ºC and the thermal conductivity of the wall material as 15W/mK. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | A 40 x 40 cm copper slab 5 mm thick at a uniform temperature of 250ºC suddenly has its surface temperature lowered to 30ºC. Find the time at which the slab temperature becomes 90ºC. Take h = 90W/m2K. | CO1 | 15 |
|  | b. | Briefly discuss about conduction shape factor. | CO1 | 5 |
| 5. |  | Air at 20oC and at atmospheric pressure flows over a flat plate at a velocity of 3 m/s. If the plate is 1 m wide and 80, calculate the following at x = 300 mm from the leading edge. a) Hydrodynamic boundary layer thickness b) Thermal boundary layer thickness c) Local friction coefficient d) Average friction coefficient e) Local heat transfer coefficient f) Average heat transfer coefficient and g) Heat transfer. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Air at 30ºC is flowing across a tube with a velocity of 25 m/s. The tube could be either a square with a side of 5 cm or a circular cylinder of diameter 5cm. Compare the rate of heat transfer in each case if the tube surface temperature is 124ºC. | CO1 | 15 |
|  | b. | Define Prandtl number and write its significance. | CO1 | 5 |
| 7. | a. | Water enters a cross flow heat exchanger (both fluids unmixed) at 5ºC and flows at the rate of 4600 kg/h to cool 4000 kg/h of air that is initially at 40ºC. Assume the overall heat transfer coefficient as 150W/m2K. For a surface area of 25m2, calculate the exit temperatures of air and water. | CO2 | 15 |
|  | b. | Classify heat exchangers. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. |  | Derive an expression for LMTD for a parallel flow and counter flow heat exchanger. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Emissivities of two large parallel plates maintained at 800ºC and 300ºC are 0.3 and 0.5 respectively. Find the net radiant heat exchange per square metre for these plates. | CO3 | 10 |
|  | b. | Explain steady state equimolar counter diffusion process. | CO3 | 10 |