Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2026 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MECHANICS OF MACHINES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain Kutzbach mobility criterion with two examples. | CO1 | 6 |
|  | b. | Determine the mobility of the following mechanism. | CO1 | 6 |
|  | c. | Calculate the degrees of freedom (mobility) of the mechanism shown in Figure. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | What is Grashoff’s Law? | CO1 | 2 |
|  | b. | A designer has developed a four-bar crank rocker mechanism with the following link lengths and claims one of the links is capable of making full revolution: i) Link 2 = 40 mm, ii)Link 3 = 100 mm iii) Link 4 = 70 mm and iv) Link 1 = 60 mm. Verify his claim using Grashoff’s Law. | CO1 | 2 |
|  | c. | Explain the following joints with neat sketches: i) Revolute joint, ii) Prismatic joint, iii) Spherical joint, iv) Cylindrical joint | CO1 | 8 |
|  | d. | What are the inversions of slider crank mechanism? Explain each of them with an example. | CO1 | 8 |
| 3. | a. | In a slider crank mechanism, as shown in figure, the crank is 40 mm long and rotates 10 rad./s. The length of the connecting rod is 120 mm. When the crank is 60° from the inner dead centre, determine the velocity of the slider and angular velocity of the connecting rod. (R and P represent revolute and prismatic joints respectively) | CO2 | 8 |
|  | b. | Angular velocity of crank OA is 600 r.p.m. Determine the linear velocity of the slider D and the angular velocity of the link BD. The dimensions of various links are : OA = 30 mm; AB = 45 mm ; BC 50 mm ; and BD = 46 mm. The distance between the centres of rotation O and C is 60 mm. The path of travel of the slider is 10 mm below the fixed point C. The slider moves along a horizontal path and OC is vertical. | CO2 | 12 |
| (OR) |  |  |  |  |
| 4. | a. | Explain velocity of rubbing in a revolute pair. | CO2 | 2 |
|  | b. | A crank , 250 mm in length, is rotating at 300 r.p.m. What is the maximum tangential velocity of the crank? What is the radial acceleration of the crank? | CO2 | 4 |
|  | c. | # The crank and connecting rod of a theoretical steam engine are 0.4 m and 1.5 m long respectively. The crank makes 300 r.p.m. in the clockwise direction. When it has turned 45° from the inner dead centre position, determine : # velocity of piston, # angular velocity of connecting rod, # velocity of point E on the mid-point of the connecting rod. | CO2 | 14 |
| 5. |  | A cam is to be designed for a knife edge follower with the following data : (i). Cam lift = 40 mm during 120° of cam rotation with simple harmonic motion. (ii). Dwell for the next 30°. (iii). During the next 120° of cam rotation, the follower returns to its original position with simple harmonic motion. (iv) Dwell during the remaining 90°. Draw the profile of the cam when the line of stroke of the follower passes through the axis of the cam shaft. The radius of the base circle of the cam is 50 mm. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A cam is to give the following motion to a roller (diameter 20mm) follower : - Outstroke during 120° of cam rotation ; - Dwell for the next 30° of cam rotation ; - Return stroke during next 120° of cam rotation, and - Dwell for the remaining 90° of cam rotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 30 mm. The follower moves with simple harmonic motion during both the outstroke and return strokes. Use the displacement diagram shown in figure. Draw the profile of the cam when the axis of the follower passes through the axis of the cam shaft. Determine the maximum velocity and acceleration of the follower during its ascent and descent, if the cam rotates at 300 r.p.m. | CO3 | 20 |
| 7. | a. | What is the functional difference between spur gear and helical gear? | CO4 | 2 |
|  | b. | A compound gear train shown in figure consists of compound gears B-C and D-E. All gears are mounted on parallel shafts. The motor shaft rotating at 900 rpm is connected to the gear A and the output shaft to the gear F. The number of teeth on gears A, B, C, D, E and F are 24, 56, 30, 90, 36 and 72 respectively. What is the train value and speed of the gear F.? A B C E D F | CO4 | 6 |
|  | c. | A pinion having 40 teeth drives a gear having 120 teeth. The profile of the gears is involute with 20° pressure angle, 12mm module and 10mm addendum. Find the length of path of contact, arc of contact and the contact ratio. | CO4 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | # Draw the profile of two teeth in a spur gear and label the following: (i) addendum (ii) pitch circle (iii) dedendum (iv) circular pitch | CO4 | 4 |
|  | b. | An epicyclic gear train consists of three gears A, B and C as shown in Fig. The gear A has 200 internal teeth and gear C has 80 external teeth. The gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 360 r.p.m. If the gear A is fixed, determine the speed of gears B and C using tabulation method. | CO4 | 16 |
|  |  | Compulsory: |  |  |
| 9. |  | A multi-plate disc clutch transmits 53 kW of power at 1500 rpm. Coefficient of friction for the friction surface is 0.16. Axial intensity of pressure is not to exceed 150 kN/m2. The internal radius is 80 mm and is 0.8 times the external radius. Find the number of plates needed to transmit required torque. Assume uniform wear. | CO5 | 20 |