Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14ME2027 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DYNAMICS OF MACHINERY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Sl. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | In the Fig (a), a slider crank mechanism is shown. A force of 3000 N is applied on the slider. Determine the forces acting on various links and also calculate the driving torque T. The linkage dimensions are: AB = 100 mm; BC = 300 mm; Angle BAC = 60º. Fig (a) | CO1 | 16 |
|  | b. | Mention the equilibrium conditions of a member with two forces and a torque. | CO1 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | A petrol engine has a stroke of 120 mm and connecting rod is 3 times the crank length. The crank rotates at 1500 rpm in clockwise direction. Determine: (i) velocity and acceleration of the piston, and (ii) angular velocity and angular acceleration of the connecting rod, when the piston has travelled one-fourth of its stroke from IDC. | CO1 | 16 |
|  | b. | Define the following terms: (i) Piston effort and (ii) Crank pin effort. | CO1 | 4 |
| 3. |  | The turning moment diagram for a multi-cylinder engine has been drawn to a scale of 1 mm to 500 Nm torque and 1 mm to 6º of crank displacement. The intercepted areas between output torque curve and mean resistance line taken in order from one end in mm2 are, -30, 410, -280, 320, -330, 250, -360, 280, -260. The engine runs at 800 rpm. The engine has a stroke of 300 mm and the fluctuation of speed is not to exceed ±2% of the mean speed. Determine a suitable diameter and cross-section of the rim of the flywheel for a limiting value of safe centrifugal stress of 7MPa and the density of the material of the flywheel is 7200 kg/m3. The width of the rim is 5 times the thickness. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A, B, C and D are four masses carried by a rotating shaft at radii 100, 125, 200 and 150 mm respectively. The planes in which the masses revolve are spaced 600 mm apart and the mass of B, C and D are 10 kg, 5 kg and 4 kg respectively. Find the required mass A and the relative angular settings of the four masses so that the shaft shall be in complete balance. | CO3 | 20 |
| 5. | a. | A shaft 50 mm diameter and 3 m long is supported at the ends and carries three loads of 1000 N, 1500 N and 750 N at 1 m, 2 m and 2.5 m from the left support. The Young’s modulus for the shaft material is 200 GN/m2. Find the frequency of transverse vibration. | CO4 | 10 |
|  | b. | The following data are given for a vibratory system with viscous damping. Mass = 2.5 kg; spring constant = 3 N/mm and the amplitude decreases to 0.25 of the initial value after five consecutive cycles. Determine the damping coefficient of the damper in the system. | CO4 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Calculate the whirling speed of a shaft 20 mm diameter and 0.6 m long, carrying a mass of 1 kg at its mid-point, The density if the shaft material is 40 Mg/m3; Young’s modulus is 200 GN/m2. Assume the shaft to be freely supported. | CO4 | 10 |
|  | b. | The measurements on a mechanical vibrating system show that it has a mass of 8 kg and that the springs can be combined to give an equivalent spring stiffness of 5.4 N/mm. The vibrating system has a dashpot which exerts a force of 40 N when the mass has a velocity of 1 m/s. Find (i) critical damping coefficient; (ii) damping factor; (iii) logarithmic decrement, and (iv) ratio of two consecutive amplitudes. | CO4 | 10 |
| 7. | a. | Derive the expression for the equivalent length of a stepped shaft. | CO4 | 4 |
|  | b. | Two rotors A and B are attached to the end of a shaft 500 mm long. The mass of the rotor A is 300 kg and the radius of gyration is 300 mm. The corresponding values of the rotor B are 500 kg and 450 mm respectively. The shaft is 70 mm in diameter for the first 250 mm; 120 mm diameter for the next 70 mm and 100 mm diameter for the remaining length. The modulus of rigidity of the shaft material is 80 GN/m2. Find: (i) The position of the node, and (ii) The frequency of torsional vibrations. | CO4 | 16 |
| (OR) |  |  |  |  |
| 8. | a. | A motor rotating at 1500 rpm drives a centrifugal pump at 500 rpm through a single stage gearing. The moments of inertia of the motor and pump impeller are 400 kg- m2 and 1400 kg- m2 respectively. The motor shaft is 45 mm in diameter and 180 mm long. The pump shaft is 90 mm in diameter and 450 mm long Determine the frequency of torsional oscillation of the system, neglecting the inertia of the gears.The modulus of rigidity of the shaft material is 84 GN/m2. | CO4 | 16 |
|  | b. | Mention the conditions that must be satisfied by an equivalent system, to find the frequency of free torsional vibrations of a geared system. | CO4 | 4 |
|  |  | Compulsory: |  |  |
| 9. | a. | The turbine rotor of a ship has a mass of 3500 kg. It has a radius of gyration of 0.45 m and speed of 3000 rpm clockwise when looking from the stern. Determine the gyroscopic couple and its effect upon the ship: - When the ship is steering to the left on a curve of 100 m radius at a speed of 36 km/h - When the ship is pitching in a simple harmonic motion, the bow falling with its maximum velocity. The period of pitching is 40 seconds and the total angular displacement between the two extreme positions of pitching is 12 degrees. | CO5 | 16 |
|  | b. | Calculate the vertical height of a Watt governor when it rotates at 60 rpm. Also find the change in vertical height when its speed increases to 61 rpm. | CO5 | 4 |